- 3GPP TSG SA WG5 Meeting #132-e	S5-204108

- August 17 – 28, 2020	revision of S5-204xxx

- Source:	Intel

- Title:	pCR 28.814 use cases and requirements for EES deployment

- Document for:	Approval

Agenda Item:	6.6.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;

# 3	Rationale

This contribution proposes to add the use cases and requirements for EES deployment to TR 28.814 [1]. The deployment of EES is defined in Figure 1 in TS 23.558 [2].

Figure 1: Architecture for enabling edge applications

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.558: "Architecture for enabling Edge Applications;".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next modified section

# 5	Use cases

## 5.1	Business level use cases

## 5.1.x	The deployment of edge enabler server(s)

#### 5.1.x.1

#### Goal

The goal is to enable the installation and termination of one or more EES(s) on the EDN (see clause 6.5.2.4 in TS 23.558 [2]). It is assumed that the ECSP and the PLMN operator can be part of the same organization.

##### 5.1.x.1.2	Description

###### 5.1.x.1.2.1	EES installation

1.	The ECSP received a request to install one or more EES(s) with deployment requirements.

2.	 The ECSP selects EDN to host the EES.

3.	The ECSP installs the EES(s).

4.	 The ECSP sends a notification to indicate that the EES(s) have been installed successfully.

###### 5.1.x.1.2.2	EES termination

1.	The ECSP received a request to terminate one or more EES(s) with the EES identifier(s).

2.	The ECSP terminates the EES(s), based on the EES identifier(s).

3. 	 The ECSP sends a notification to indicate that the EES(s) have been terminated successfully.

#### 5.2	Specification level use cases

Editor’s note: this clause is to describe the use cases associated with the management services to support orchestration and management of edge computing.

### 5.2.x	EES Instantiation

#### 5.2.x.1

#### Goal

The goal is to enable the installation of one or more EES(s) on the EDN (see clause 6.5.2.4 in TS 23.558 [2]).

#### 5.2.x.2	Description

An operator would like to deploy its EDN as local data network. It starts with defining the serving location and constituent edge entities (EAS and EES) for each EDN. The 3GPP management system is requested to instantiate the required entities, as 3GPP network functions, aiming to server the particular location. The instantiated EES may server one or multiple EAS.

1.	A consumer request for EES(s) instantiation providing EES deployment requirements.

2.	The 3GPP management system determines the EDN where the EES(s) will be installed.

3.	The 3GPP management system performs the following operations to install the EES:

-	Instantiate the EES VNF(s).

-	Connect the EES(s) to the 5GC network functions.

Note: Connecting EES with 5GC network function will enable information exchange over appropriate interfaces (e.g N6, EDGE-7).

4.  The 3GPP management system sends a notification to indicate that the EES(s) have been installed successfully.

###### 5.2.x.3	Requirements

REQ-EAS-DEPLOY-FUN-1: 3GPP management service producer should have the capability to instantiate the EES, as per request from consumers.

REQ-EES-DEPLOY-FUN-2: 3GPP management service producer should have the capability to send the notification indicating the status of EES installation.

REQ-EES-DEPLOY-FUN-3: 3GPP management system producer should have the capability to relate instantiated EES with one or multiple EAS which it will be serving.

### 5.2.y	EES Termination

#### 5.2.y.1	Goal

The goal is to enable the termination of one or more EES(s) on the EDN (see clause 6.5.2.4 in TS 23.558 [2]).

##### 5.2.y.2	Description

1. 	A consumer consumes the management service of 3GPP management system to terminate the EES with the EES identifier.

2. The 3GPP management system terminates the EES VNF based on the EES identifier, and disconnects the EES from the 5GC network functions.

3. The 3GPP management system sends a notification to indicate that the EES has been terminated successfully.

###### 5.2.x.1	Requirements

REQ-EES-DEPLOY-FUN-1 3GPP management service producer should have the capability to terminate the EES with the EES identifier, as per request from consumers

REQ-EES-DEPLOY-FUN-2 3GPP management service producer should have the capability to send the notification indicating the status of EES termination.

End of modified section