- 3GPP TSG-SA5 Meeting #134e	S5-206088rev1

- 16 to 25 November 2020, E-meeting

- Source:	Intel

- Title:	pCR 28.809 Update solutions on request of Management Data Analytics Report

- Document for:	Approval

- Agenda Item:	6.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Management and orchestration; Study on enhancement of Management Data Analytics (MDA)”

# 3	Rationale

The use case and solutions on request of Management Data Analytics Reports are described in TR 28.809 [1]. The streaming data reporting service and file reporting service defined in TS 28.532 are suitable for Management Data Analytics Reports reporting. And, the streaming data reporting service has already preserved to support streaming of analytics reporting since Rel-16.

However, it is not clearly described in solutions that how the request of Management Data Analytics Reports is supported by the streaming data reporting service and file reporting service.

This pCR is to update the solutions on request of Management Data Analytics Report in connection with the generic streaming data reporting service and file reporting service defined in TS 28.532.

# 4	Detailed proposal

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[x]	3GPP TS 28.532: "Management and orchestration; Generic management services".

Next Modified Section

### 6.99.3	Retrieval of Management Data Analytics Reports

#### 6.99.3.1	Use case

Besides the subscription and reporting of the MDA reports, the consumer may need to retrieve some historical MDA reports.

The consumer submits a request to MDAS producer to retrieve the historical MDA reports. This request may include a time frame and a filter to specify the scope of MDA reports to be retrieved (e.g, type of analytics report such as coverage issue analysis, resource utilization analysis, the managed functions to be analysed, etc.).

The MDAS producer retrieves the historical MDA reports, and sends the results to the consumer using the same reporting method designated in the subscription.

#### 6.99.3.2	Potential requirements

REQ-MDA_REQ-1		The MDAS producer should have a capability to allow an MDAS consumer to request a  historical analytics report. The request should optionally allow the MDAS consumer to filter the scope of data in the analytics report.

REQ-MDA_REQ-2		The MDAS producer should have a capability to provide the retrieved historical analytics report to the MDAS consumer.

#### 6.99.3.3	Possible solutions

The MDAS consumer sends the MDAReportRetrieval request to MDAS producer, with the following information included:

-	identifier of the retrieval request;

-	time frame within which the MDA reports are to be retrieved;

-	filter for the scope of the MDA report (e.g., type of analytics report, managed functions to be analysed, etc.)

The MDAReportRetrieval may be modelled as an IOC and managed via provisioning related operations (such as CreateMOI, ModifyMOI, DeleteMOI), or sent by a dedicated operation (separated from the provisioning related operations).

The MDAS producers provides a response indicating the status of the request.

For the MDA report subscription designating the reporting method of file reporting, the possible solution in connection with file reporting service defined in TS 32.532 [x] is shown in Figure 6.99.3.3-1.

Figure 6.99.3.3-1: MDA report retrieval based on MDA report file reporting

For the MDA report subscription designating the reporting method of streaming data reporting, the possible solution in connection with streaming data reporting service defined in TS 32.532 [x]  is shown in Figure 6.99.3.3-2.

Figure 6.99.3.3-2: MDA report retrieval based on MDA report streaming

#### 6.99.3.4	Conclusion

The solution described in clause 6.99.3.3 reuses the generic streaming data reporting service and file reporting service defined in TS 28.532 [x] for reporting the retrieved MDA data, and requires to a mechanism for MDA report retrieval that can be defined in the normative work.

Therefore, this solution is a feasible candidate for MDA report retrieval.

End of Modified Sections