3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211195

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.130 | CR | 0006 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the business requirements to support NG-RAN sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS |  |  |  |  |  | Date: |  |  | 2021-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NG-RAN sharing is not included in the business requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NG-RAN in the supported radio access technologies. - Add the description of the roles for NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The business requirements for NG-RAN sharing will not be included. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of Changes

5	Business level requirements

5.1	Requirements

5.1.1	Requirements for the OAM&P of shared RAN

REQ-NS_GEN-CON-1		The MOP–SR-DM shall support a capability allowing the MOP-NM to manage S-RAN of any of the following radio access technologies:

i/ GERAN;

ii/ UTRAN;

iii/ E-UTRAN;

iV/ NG-RAN.

REQ-NS_GEN-CON-2		The MOP–SR-DM shall support a capability allowing the MOP-NM to manage S-RAN according to any of the following scenarios:

i/ Multi-Operator Core Network

ii/ Gateway Core Network.

REQ-NS_GEN-CON-3		The MOP–SR-DM shall support a capability allowing the MOP-NM to know which POPs the shared resources belong to.

REQ-NS_GEN-CON-4		The MOP–SR-DM shall support a capability allowing the MOP-NM to configure which POPs share each cell.

REQ-NS_GEN-CON-5		Any POP shall be able to activate a signaling-based trace / MDT session on its subscribers, whether they are attached to the POP own RAN or to the S-RAN, provided:

a/ this is compliant with the RAN sharing contract; and

b/ user consent for participation in MDT is respected.

REQ-NS_GEN-CON-6		The MOP shall be able, on behalf of any POP, to activate an area-based trace / MDT session, on the portion of the S-RAN that the POP shares and on the POP subscribers only, from the MOP RAN DM or MOP NM, provided

a/ only the POP related subscriber data are collected;

b/ this is compliant with the RAN sharing contract; and

c/ user consent for participation in MDT is respected.

REQ-NS_GEN-CON-7		The MOP shall be able, on behalf of multiple POPs, to activate an area-based trace / MDT session, on the portion of the S-RAN that each POP shares and on each POP subscribers only, from the MOP RAN DM or MOP NM, provided:

a/ each POP has access only to its subscriber data (i.e. not to other POPs' subscriber data);

b/ this is compliant with the RAN sharing contract; and

c/ user consent for participation in MDT is respected.

5.1.2	Requirements for the OA&M of shared core network elements

REQ-NS_GEN-CON-8			In the GWCN scenario, the MOP shall be able to manage S-CORE.

REQ-NS_GEN-CON-9			The MOP shall be able to know which POPs the shared core network elements belong to.

REQ-NS_GEN-CON-10		The MOP shall be able to configure which POPs share each core network element.

5.1.3	Requirements for the management of measurements for cross-operator accounting based on data volume and QoS

REQ-NS_PM-CON-1	The MOP shall be able to charge the POPs for the data volume used by POP’s users per selected QoS profile criteria via measurements defined for shared networks.

-	The QoS profile criteria may include QCI Indicator, GBR Indicator, ARP Indicator.

-	It shall be possible to differentiate between DL and UL.

REQ-NS_PM-CON-2	The MOP shall be able to set the reliability for the selected measurements defined for cross operator accounting purpose in shared networks.

REQ-NS_PM-CON-3	A maximum number of 200 counter instances (for measurements defined for cross operator accounting purpose in shared networks) can be recorded per granularity period.

5.2	Actor roles

For GERAN, UTRAN, and E-UTRAN:

MOP-SR-DM:		An entity performing an IRPAgent role in MOCN and in GWCN.

MOP-SC-DM:		An entity performing an IRPAgent role in GWCN.

MOP-NM:			An entity performing the IRPManager role in MOCN and in GWCN.

For NG-RAN:

MOP-SR-DM:		An entity contains domain management Service Providers and Service Consumers in MOCN.

MOP-NM:			An entity contains network management Service Providers and Service Consumers in MOCN.

End of Change