3GPP TSG SA WG5 Meeting 137-e		S5-213391r02

- e-meeting, 10th - 19th May 2021					                     							revision for S5-21xxxx

- Source:	Alibaba group

- Title:	Add Description of Concept and Roles to TR 28.824

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR specifies the concepts and roles related to network management capability exposure.

# 4	Detailed proposal

Start of 1st Change

# 4	Overview

## 4.X	General

### 4.X.1	Concepts related to network management capability exposure

### 4.X.1.1	Network management capability exposure

Network management capability exposure means that an external MnS consumer which is outside 3GPP management system can conditionally consume the MnS within 3GPP management system via BSS. Network management capability exposure may rely on a dedicated MnF (e.g. EGMF) that manages the exposure aspects. Alternatively, the MnFs in which certain MnSs are required by external MnS consumer can also manage the exposure aspects by themselves. Depending on specific use cases, MnS associated to 5G NRM fragments (e.g. NR, 5GC, network slice) can be possibly exposed to certain external MnS consumer under certain condition and permission.

### 4.X.2	Roles related to network management capability exposure

### 4.X.2.1	Network management capability exposure consumer

One of the roles related to network management capability exposure (NMCE) is NMCE consumer which can be external consumer outside 3GPP management system. The NMCE may be related to MnS within 3GPP management system, which is required to be access by  NMCE consumer (e.g. CSC or NSC). An example of NMCE can be verticals (i.e. Industry, Internet Company, etc). The NMCE consumers usually have specific service requirement on 5G network for their applications and wish to get access to certain MnSs within 3GPP management system. In order to enable the NMCE consumer for the access of certain MnSs, the stakeholder (e.g. MNO, MVNO) usually makes contract with the vertical which is the NMCE consumer. The contract may contain the agreement on what MnS, optionally under what condition, can be exposed. The contract can be made using an online check list which is available through BSS. Alternatively, the contract can be made offline between stakeholders. The condition can be certain constraint of MnS access based on the contract, e.g. the access quota of certain MnS, the access frequency of certain MnS, etc.

### 4.X.2.2	Network management capability exposure producer

Another role that is related to the network management capability exposure is the NMCE producer which can be MnS producer that is required to be consumed by NMCE consumer. NMCE producer can be CSP or NOP which has the capability to allow the access of certain MnS from the externals (e.g. verticals and service provider) outside the 3GPP management system. Once the vertical makes the contract with the stakeholder (e.g. MNO, MVNO) regarding exposing certain MnSs. The NMCE producer can expose certain MnSs that are allowed to be accessed by the MNCE consumer through BSS. The contract can include what kinds of MnS, optionally under certain condition, can be exposed to the NMCE consumer and the corresponding permission is configured on certain MnFs within 3GPP management system.

End of Change