3GPP TSG SA WG5 Meeting 137-e		S5-213391r03

- e-meeting, 10th - 19th May 2021					                     							revision for S5-21xxxx

- Source:	Alibaba group

- Title:	Add Description of Concept and Roles to TR 28.824

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR specifies the concepts and roles related to network management capability exposure.

# 4	Detailed proposal

Start of 1st Change

# 4	Overview

## 4.X	General

### 4.X.1	Concepts related to network management capability exposure

### 4.X.1.1 Exposure of Management Services

Exposure of management services indicates the case that an external MnS consumer which is outside 3GPP management system can conditionally consume the MnS within 3GPP management system via BSS. MnS exposure service represents the MnS that can be exposed by MnS producer to the external consumer via BSS. MnS exposure service may rely on a dedicated MnF (e.g. EGMF) that manages the exposure aspects. Alternatively, the MnFs in which certain MnSs are required by external MnS consumer can also manage the exposure aspects by themselves.

### 4.X.2	Roles related to network management capability exposure

### 4.X.2.1	MnS exposure service consumer

One of the roles related to exposure of management services is MnS exposure service consumer which can be external consumer outside 3GPP management system. The MnS exposure service may be correlated with MnS within 3GPP management system, which is required to be access by MnS exposure service consumer that can be associated to CSC or NSC. An example of MnS exposure service consumer can be verticals (i.e. Industry, Internet Company, etc). The MnS exposure service consumers usually have specific service requirement on 5G network for their applications and wish to get access to certain MnSs within 3GPP management system. In order to enable the MnS exposure service consumer for the access of certain MnSs, the stakeholder (e.g. MNO, MVNO) usually makes contract with the vertical which is the MnS exposure service consumer. The contract may contain the agreement on what MnS, optionally under what condition, can be exposed. The contract can be made using an online check list which is available through BSS. Alternatively, the contract can be made offline between stakeholders. The condition can be certain constraint of MnS access based on the contract, e.g. the access quota of certain MnS, the access frequency of certain MnS, etc.

### 4.X.2.2	MnS exposure service producer

Another role that is related to exposure of management service is the MnS exposure service producer which can be MnS producer that is required to be consumed by MnS exposure service consumer. MnS exposure service producer can be associated to CSP or NSP which has the capability to allow the access of certain MnS from the externals (e.g. verticals and service provider) outside the 3GPP management system. Once the vertical makes the contract with the stakeholder (e.g. MNO, MVNO) regarding exposing certain MnSs. The MnS exposure service producer can expose certain MnSs that are allowed to be accessed by the MnS exposure service consumer through BSS. The contract can include what kinds of MnS, optionally under certain condition, can be exposed to the MnS exposure service consumer and the corresponding permission may be configured on certain MnFs within 3GPP management system.

End of Change