3GPP TSG SA WG5 Meeting 137-e		S5-213392r03

- e-meeting, 10th - 19th May 2021					                     							revision for S5-21xxxx

- Source:	Alibaba group

- Title:	use case – network slice management capability exposure

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR is to study the use case on network slice management capability exposure.

# 4	Detailed proposal

Start of 1st Change

## 5.Y	Network slice management capability exposure

### 5.Y.1	Description

A use case of network slice management capability exposure can be described as follows:

1. MNO A selects the MnS that can be exposed to Externals and publishs the related MnS data using a proper discovery service for exposure. The discovery service for exposure allow the Externals to figure out what kind of MnSs are allowed to be exposed.

2. In order to obtain the network slice management capability exposure, a vertical A (i.e. MnS exposure service consumer) firstly have a contract with an MNO A. During the negotiation of the contract, the vertical A negotiates its specific requirements for the network slice management capability exposure to the MNO. The negotiation can be done via the following ways:

a) The MnS exposure service consumer A can use the online check list for the network slice management capability exposure provided by the MNO. The check list can be available through the BSS or transparently displayed via BSS.

b) Alternatively, the MnS exposure service consumer can negotiate the contract and the related specific requirements on the network slice management capability exposure offline (e.g. through a F2F meeting).

3. The MNO can upload the contract with the requirement of the network slice management capability exposure to the BSS. The BSS may interact with the OSS in order to generate permission rule for access MnS based on the customized requirement from the MnS exposure service consumer.

4. After the generation of the permission rules, the permission rules can be configured in the OSS domain, which take the responsible for managing the permission for all the MnSs within the 3GPP management system. The configured permission rule will take effect when the corresponding MnFs receive the requests from the MnS exposure service consumer, e.g. to read maxNumberofUEs within NetworkSlice, etc.

### 5.Y.2	Issue and gaps

#### Gap:

The definition and the format of the permission rule for network management capability exposure of network slice related managed object is not specified in existing 3GPP management system.

How to configure the permission rule for network management capability exposure of network slice related managed object in 3GPP management system is not specified in current specifications.

How to publish MnS which can be exposed to externals to a suitable MnS discovery service producer for network management capability exposure is not specified in existing 3GPP management system.

How to manage the published MnS data of a MnS discovery service producer for network management capability exposure is not specified in existing 3GPP management system.

Whether a discovery permission for network management capability exposure is needed is not well studied in existing 3GPP management system.

End of Change

Start of 2nd Change

- 6	Potential requirement for network management capability exposure

- REQ-NSCE-x The 3GPP management system should have the capability to support the configuration of permission rules for exposing network slice related managed object.

- REQ-NSCE-x The 3GPP management system should have the capability to publish MnS which can be exposed to externals to a suitable MnS discovery service producer for network management capability exposure.

- REQ-NSCE-x The 3GPP management system should have the capability to manage the published MnS data of a MnS discovery service producer for network management capability exposure.

- REQ-NSCE-x The 3GPP management system should have the capability to support the configuration of discovery permission for network management capability exposure if needed.

End of Change