3GPP TSG SA WG5 Meeting 137-e		S5-213658d1

- e-meeting, 10th - 19th May 2021					                     						   revision for S5-213392

- Source:	Alibaba group

- Title:	use case – network slice management capability exposure

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR is to study the use case on network slice management capability exposure.

# 4	Detailed proposal

Start of 1st Change

## 5.Y	Network slice management capability exposure

### 5.Y.1	Description

A use case of network slice management capability exposure can be described as follows:

1. MNO A selects the MnS that can be exposed to Externals (e.g. verticals) and publishes the related MnS data in certain MnS producer that the BSS can visit. Externals can obtain the MnSs that are allowed to be exposed via BSS (e.g. through Service Catalog).

2. In order to enable the expose of network slice related MnS, a vertical A firstly makes a contract with the MNO A. The vertical A negotiates its specific requirements for the network slice management capability exposure with the MNO. The negotiation can be done via the following ways:

a) For vertical A which is small enterprise, it can directly have a view on the network slice related MnSs that are available to be exposed through the BSS (e.g. by using Service Catalog). Based on that, the vertical A can select the network slice related MnSs which will be covered by the contract.

b) For vertical A which is large enterprise (i.e. Internet giants that have their own service customer), it can select the network slice related MnSs that are available to be exposed offline (e.g. through a F2F meeting).  The MNO can proceed with the service ordering through BSS based on the contract.

3. The BSS may interact with the OSS in order to complete certain configuration (i.e. permission regarding what MnS, optionally under what condition, can be exposed) regarding the exposure of MnS based on the customized requirement from the eMnS consumer (i.e. vertical A).

4. The vertical A can get access to the network slice related MnSs through BSS (e.g. through Service Catalog).

### 5.Y.2	Issue and gaps

#### Gap:

The definition and the format of permission for the exposure of network slice related MnS and its potential impact on BSS is not discussed in current SA5 work.

Whether and how to publish MnS which can be exposed to BSS to a suitable MnS producer for network management capability exposure is not specified in existing 3GPP management system.

End of Change