3GPP TSG SA WG5 Meeting 137-e		S5-213658d3

- e-meeting, 10th - 19th May 2021					                     						   revision for S5-213392

- Source:	Alibaba group

- Title:	use case – network slice management capability exposure

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR is to study the use case on network slice management capability exposure.

# 4	Detailed proposal

Start of 1st Change

## 5.Y	Network slice management capability exposure

### 5.Y.1	Description

A use case of network slice management capability exposure can be described as follows:

1. MNO A selects the MnS that can be exposed to Externals (e.g. verticals) and publishes the related MnS data in certain MnS producer that the BSS can access.

2. In order to enable the exposure of network slice related eMnS, a vertical A firstly makes a contract with the MNO A. The vertical A negotiates its specific requirements for the network slice management capability exposure with the MNO A. The negotiation can be done via the following ways:

a) For vertical A which is small enterprise, it can directly have a view on the network slice related management capability through the BSS (e.g. by using Service Catalog). Based on that, the vertical A can select the network slice related eMnSs which will be covered by the contract.

b) For vertical A which is large enterprise (i.e. Internet giants that have their own service customer), it can select the network slice related eMnSs that are available to be exposed offline (e.g. through a F2F meeting). The MNO A can proceed with the service ordering through BSS based on the contract.

3. The BSS may interact with the OSS in order to complete certain configuration (i.e. permission regarding what eMnS, optionally under what condition, can be exposed) regarding the exposure of eMnS based on the customized requirement from the eMnS consumer (e.g. vertical A).

4. The vertical A can get access to the network slice related management capability offered by eMnS producer within 3GPP management system. The access may need the interaction with BSS (e.g. through Service Catalog).

### 5.Y.2	Issue and gaps

#### Gap:

An External needs to apply for the access of network slice management capability through BSS. However, there is no discussion and agreement on whether an eMnS is exposed transparently through the BSS or being processed through a dedicated exposure platform before exposing to the external.

The definition and the format of permission for the exposure of network slice related eMnS and its potential impact on internal interface with BSS is not discussed in current SA5 work.

Whether and how to publish eMnS which can be exposed to BSS to a suitable eMnS producer for network management capability exposure is not specified in existing 3GPP management system.

End of Change