3GPP TSG-SA5 Meeting #141-e 	S5-221079

e-meeting, 17 -26 January 2022

Title:	Reply LS on slicing management aspects in relation to SEAL

Response to:	[S6-210709] LS on slicing management aspects in relation to SEAL

Release:	Release 17

Source:	3GPP SA5

To:	3GPP SA6

Cc:	3GPP SA2, SA

Contact Person:

Name:	Kai ZHANG

E-mail Address:	kai.zhangkai@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA5 would like to thank SA6 for their LS on slicing management aspects in relation to SEAL.

SA5 provides the following feedback on the SA6 LS:

- Views of SA5 about the exposure of slicing management aspects to SEAL (Service Enabler Architecture Layer for Verticals - TS 23.434) as a means to address requirements from third party applications

  - SA5 specifies 5G network management capabilities including:

    - Network Element / Network Function management, e.g. RAN and CN NE/NF management

    - Network management (including network slice management in a scenario such as ‘NOP internals’ – see 3GPP TS 28.530),

    - Service management (including network slice management in a scenario such as ‘Network Slice as a Service’ (NSaaS) – cf. 3GPP TS 28.530).

and Charging management for 5G.

  - 3GPP 5G Management Services (MnS) offered by the 5G management system act on network elements / network functions, networks and services, and may be exposed to external consumers. The 3GPP exposure governance management function (EGMF) is in charge of management service exposure governance. 3GPP MnS consumers may be in the Network Operator (NOP) domain or outside the NOP domain (e.g. in the Vertical domain).

  - 3GPP SA5 is responsible for the specification of the 5G Management and Charging Services, as well as for the specification of the access control and exposure of 5G management services.

  - To fulfil the management capabilities exposure requirements from third party applications which are outside the NOP domain, SA5 management capability exposure governance feature (see TS 28.533) uses EGMF to expose management capabilities to external third party applications acting as AF defined in TS 23.501.

- Views of SA5 about interactions with ongoing work in SA5, in particular

  - Exposure of management capability in the context of non-public networks (see TS 28.557) is about access to management services provided by an NPN-SP (NPN Service Provider) and accessed by an NPN-SC (NPN Service consumer). The external third party applications may be acting as one kind of such external NPN-SC consumer.

  - FS_MNSAC is about authentication and authorization of an internal or external consumer that wants to access the management services (MnS) provided by an MnS producer. The external third party applications may be acting as one kind of such external consumer.

- SA5 agrees with SA6 as the highlighted need to coordinate with each other WG to avoid overlapping specifications and ensure complementary approaches.

2. Actions:

To SA6 group:

ACTION: 	SA5 respectfully requests SA6 to take this information into account.

3. Date of Next TSG-SA WG5 Meetings:

SA5#142e	04 - 12 April 2022	Electronic meeting

SA5#143e	09 - 17 May 2022		Electronic meeting