3GPP TSG-SA5 Meeting #142-e 	S5-222379rev1

- e-meeting, 4-12 April 2022

- Source:	China Telecom, China Mobile, HUAWEI

- Title:	pCR 28.104 Add MDA capability for MDA assisted energy saving analysis

- Document for:	Approval

- Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104-100 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This pCR is to add the definition of trafficThrshold defined in clause 8.5.6.2 for TS 28.104 [1].

# 4	Detailed proposal

Start of modifications

### 8.5.6	EsRecommendationsOnNRcell <<dataType>>

#### 8.5.6.1	Definition

This data type specifies the type of energy saving recommendations on NR cells.

#### 8.5.6.2	Information element

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EsNRcells | It provides the DN of NR cells (ES-Cell) which are recommended to enter energySaving state. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CandidateNRcells | It provides the DN of candidate NR cells which are recommended with precedence for taking over the traffic of ES-Cell. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnterTime | It provides the recommended time to enter the energy saving state for the ES-Cell. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTime | It provides the recommended time to terminate the energy saving state for the ES-Cell | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficThresholds | It provides the recommended traffic threshold information. The ES-Cell can enter the energy saving state when the traffic is below the threshold value defined in the thresholdValue. | M | type: ThresholdInfo (see TS 28.622) multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modifications