3GPP TSG-SA5 Meeting #142-e 	S5-222462rev2

- e-meeting, 4 - 12 April 2022

- Source:	Huawei

- Title:	PCR TS 28.104 add mobility performance analysis solution

- Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	SP-210132 Enhancements of Management Data Analytics Service

[2]	3GPP TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This contribution propose to add mobility performance analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AI				Artificial Intelligence

MDA			Management Data Analytics

MDAS		Management Data Analytics Service

MDA MnS	MDA Management service

ML			Machine Learning

CHO	           Conditional Handover

DAPS	           Dual Active Protocol Stack

2nd modified section

### X.X.1	MDA assisted mobility management

#### X.X.1.X		Mobility Performance analysis

##### X.X.1.X.1	MDA type

The MDA type for Mobility Performance analysis is: MobilityManagementAnalytics. MobilityPerformanceAnalysis.

##### X.X.1.X.2	Enabling data

The enabling data for mobility performance analysis are provided in table X.X.1.X.2-1.

For general information about enabling data, see clause 8.2.1.

Table X.X.1.X.2-1: Enabling data for Mobility Performance analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Inter-gNB handovers | Inter-gNB handovers (clause 5.1.1.6.1 of TS 28.552 [4]). |
|  | Intra-gNB handovers | Inter-gNB handovers (clause 5.1.1.6.4 of TS 28.552 [4]). |
|  | Inter-gNB DAPS handovers | Inter-gNB handovers (clause 5.1.1.6.2 of TS 28.552 [4]). |
|  | Intra-gNB DAPS handovers | Inter-gNB handovers (clause 5.1.1.6.3 of TS 28.552 [4]). |
|  | Inter-gNB conditional handovers | Inter-gNB handovers (clause 5.1.1.6.6 of TS 28.552 [4]). |
|  | Intra-gNB conditional handovers | Inter-gNB handovers (clause 5.1.1.6.7 of TS 28.552 [4]). |

##### X.X.1.X.2	Analytics output

The specific information elements of the analytics output (MDA report) for mobility performance analysis, in addition to the common information elements of the analytics outputs (see clause X.Y), are provided in table X.X.1.X.2-1.

Table X.X.1.X.2-1: Analytics output for Mobility Performance analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| mobilityPerformance IssueIdentifier | The identifier of the mobility performance issue analysis; | M | type: interge multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mobilityPerformance IssueRootCause | The root cause of mobility performance issues. The allowed value is one of the enumerated values: too long mobility interruption time, poor coverage of the cell-edge, inappropriate handover parameters, other. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mobilityPerformance IssueLocation | Geographical location areas where the mobility performance issue occurred. | O | type: GeoArea (see TS 28.622, to be confirmed) multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modified section