3GPP TSG-SA5 Meeting #142-e 	S5-222214

# e-meeting, 4 - 12 April 2022

# Source: Nokia, Nokia Shangail Bell

# Title:  RequestedUnit type correction

# Document for: Approval

Agenda Item: 7.5.2

# 1	Decision/action requested

Please approve

# 2	References

[1]	3GPP TR 28.826 V0.3.0 Study on Nchf charging services phase 2 improvements and optimizations

# 3	Rationale

There is a need to provide accuracy on the intial Service charging. ServiceID is used in UsedUnitContainer, though its not included in RequestedUnit.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 32.255: "5G data connectivity domain charging; stage 2".

[3]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3"

Second change

# 4	Overview

The Nchf converged charging services was first released in Rel-15. This first iteration was to a large extent, with some minor exceptions, copying and combining its predecessors, Ro and Rf, based on Diameter to a service based interface (SBI) supporting converged charging service. The depiction of the internal architecture of the Converged Charging System (CCS) was to a large extent based on the Online Charging System (OCS).

The combination of the Diameter Ro and Rf interfaces into one converged charging service together with the move of the integration point for CDRs have led to that all the information previously reported and transported over two or more interfaces now needs to be transported over a single interface. This makes it essential that the converged charging interface is optimized in regard to the amount of information and number of reports sent, or at least allows for such an optimization if needed.

Furthermore, there is a need to improve the way rating and quota is granted for a specific service. In order to be accurate on the rating, service id shall be known from the initial Charging Data request. Ultimately, this will facilitate operations like service charging enhancement, reporting and the monetization either by the Service Provider or Tenant.

Low latency requirements increased by new business cases may need enhancements to the non-blocking mechanism.

Charging of new service together with converged charging have increased the need for more information in the input to rating.

The use of event-based charging has led to a need for cancelling the charging for the chargeable event, e.g. refund, for unsuccessful scenarios or cases not possible to fulfil.

The document structure of the actual service, i.e. yaml, followed the handling of Ro and Rf, based on Diameter, which may need improvements.

Third change

## Rating input enhancement

### 5.3.1	General

### 5.3.2	Use cases

#### 5.3.2.1	Use Case #3a: Enhancement of input to CHF rating

A End User (subscriber) has subscribed to a service with multiple QoSs. The PCF dynamically decides to authorize a QoS based on the network information. When granting a quota, the CHF may need additional authorized QoS information corresponding to the Rating Group to assist CHF to grant quota per Rating Group.

As per the current specifications, the PDU charging information includes the default QoS information (Authorized QoS Information for PDU session and Subscribed QoS Information), which there is not the QoS information corresponding to the Rating Group.

The QoS information is an example as the input to the CHF rating.

Editor’s note: The other information for the enhancement of input to CHF rating will be introduced case by case and ffs.

#### 5.3.2.2	Use Case #3d: Service Charging Enhancement

An End User (subscriber) subscribed to a service, once the service consumption is authorized for the End User, CHF can provide the right Rating Group to be used for service charging whilst the End User consumes it. Furthermore, this can be used for a clear improvement on the charging statistics and reporting.

As per the current specifications, the Charging Data Request [intial] message has the RequestedUnit Type which will be used by the CHF on the response, though the ServiceID type is not available. Service ID is only available in the UsedUnitContainer type (after intitial Chargind Data Request message).

### 5.3.3	Potential charging requirements

REQ-3GPPCH-ER-01	The 5G system should support the enhancement of input to CHF rating based on the QoS information.

REQ-3GPPCH-ER-02	The 5G system should support the enhancement of input to CHF rating based on the Service ID information.

### 5.3.4	Key issues

The following key issues are identified:

-	Key Issue #3a: Identification and classification of information which can be used as the input to CHF rating.

-	Key Issue #3b: Identify the Network Functions which can provide the input to support the CHF rating.

-	Key Issue #3c: Determine the interaction to support the enhancement of input to CHF rating.

-	Key Issue #3d: Identify the interaction to support the option of providing the Service ID on the initial Charging Data Request.

### 5.3.5	Solutions

In order to facilitate the rating on an initial charging request, the change on the Requested Unit type is propose in TS 32.291 Table 6.1.6.2.1.9-1 [5] as indicated below.:

Table 6.1.6.2.1.9-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceId | ServiceId | OC | 0..n | This field identity of the used service |  |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| NOTE 1: f none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

End of changes