3GPP TSG-SA5 Meeting #146Bis-e 	S5-231069rev1

Electronic meeting, 16 - 19 January 2023

- Source:	Huawei

- Title:	Evaluation and conclusion of clause 7.1

- Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

For adding evaluation and conclusion of clause 7.1

# 4	Detailed proposal

First change

### 7.1.5	Evaluation

For key issue#1a, covering requirements REQ-CH_VMNO-01 and REQ-CH_VMNO-02,

-	solution #1.1 (V-SMF interacts with V-CHF) is applicable to Home Routed and Local Breakout, and has been covered by existing specifications;

-	solution#1.3 (V-SMF interacts with V-CHF and H-CHF) is applicable to local breakout and is same with solution#2.2 which is covered in R17;

-	solution#1.9 (V-SMF interacts with V-CHF by two session) is applicable to local breakout, based on the architecture of solution 2.1. However, solution #2.1 includes procedure of single session, solution #1.9 includes procedure of two sessions. Two session procedure needs more signaling interaction and is more complex, single session procedure cost less than two session procedure.

For key issue#1a, covering requirements REQ-CH_VMNO-05,

-	solution#1.4 (Roaming Charging Profile exchanged between HPLMN and VPLMN) is applicable for Home Routed, needs necessary clarification "The V-SMF reports the updated "Roaming Charging Profile" to V-CHF in the next Charging Data Request[Update] which is armed by subsequent triggers" on current specification.

For key issue#1b, covering REQ-CH_VMNO-03 and REQ-CH_VMNO-04,

-	solution#1.2 (V-AMF interacts with V-CHF) is applicable to Local breakout, this solution has been covered by existing specifications.

-	solution#1.6 (V-AMF interacts with V-CHF and H-CHF) is applicable to local breakout, this solution has been covered by existing specifications.

For key issue#1d, covering REQ-CH_VMNO-01 and REQ-CH_VMNO-02,

-	solution 1.8 (Only QBC trigger negotiation towards the V-CHF) is applicable to local breakout scenario.

For key issue#1e, covering REQ-CH_VMNO-01 and REQ-CH_VMNO -02,

-	solution 1.11(QoS flow level trigger Setting during PDU session) is applicable to local breakout and home routed scenarios.

For key issue#1f, covering REQ-CH_VMNO-01 and REQ-CH_VMNO -02,

-	solution 1.10 (Separated PDU Session level setting) is applicable to local breakout and home routed scenarios. With this solution, PDU session level trigger setting for QBC and PDU session level trigger setting for FBC have no impact on each other.

### 7.1.6	Conclusion

No normative work needs to done based on solutions #1.1, #1.2, #1.3 and #1.6.

The Home Routed solution enhancement solution #1.4 needs to be taken into normative work.

For trigger setting in local breakout and home routed scenarios, solutions #1.10 (QoS flow level trigger Setting during PDU session) and #1.11 (QoS flow level trigger Setting during PDU session) need to be taken into normative work.

For trigger setting in local breakout scenario, solution #1.8 (Only QBC trigger negotiation towards the V-CHF) needs to be taken into normative work.

End of change