3GPP TSG SA WG5 Meeting #148-e 	S5-233421rev2

Electronic, 17-25 April 2023

- Source:	Huawei

- Title:	Add Solution #2.x for Charging session between V-CHF and H-CHF per charging session between NF and V-CHF

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed solution in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

To provide an alternative solution for handling the charging session between V-CHF and H-CHF. Add a new key issue that specifies the need to handling the granularity of charging session between visited CHF and home CHF.

# 4	Detailed proposal

First change

### 7.2.3	Key issues

The following key issues are identified:

-	Key Issue #2a: Collection of charging mechanism in visited MNO about the 5G data connectivity usage.

-	Key Issue #2b: Conveying the charging information to the home MNO for 5G data connectivity usage.

- 	Key Issue #2c: Finding the correct CHF

-	Key Issue #2d: Service based interface to use between visited CHF and home CHF

-	Key Issue #2e: Trigger handling between visited CHF and home CHF for 5G data connectivity usage.

-	Key Issue #2f: Collection of charging mechanism in visited MNO for SMS usage.

-	Key Issue #2g: Conveying the charging information to the home MNO for SMS usage

-	Key Issue #2h: Collection of charging mechanism in visited MNO for 5G connection and mobility.

-	Key Issue #2i: Conveying the charging information to the home MNO for 5G connection and mobility.

-	Key Issue #2j: The granularity of charging session between visited CHF and home CHF.

### 7.2.4	Potential solutions

#### 7.2.4.x	Solution #2.x: Charging session between V-CHF and H-CHF per charging session between NF and V-CHF

##### 7.2.4.x.1	General

A possible solution for key issue #2j, would be to have the charging session between V-CHF and H-CHF with the same granularity as the charging session between NF(CTF) and V-CHF. In specific, V-CHF establishes a charging session with H-CHF when it receives a charging data request from NF(CTF) for session establishment, and releases the charging session with H-CHF when it receives a charging data request from NF(CTF) for service release.

##### 7.2.4.x.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.x.3	Message flows

In this case, the SMF, AMF, and SMSF establish the charging session and events with the V-CHF per as specified in the TS 32.255 [4], TS 32.256 [5], and TS 32.274 [8]. The V-CHF would establish the charging session with the H-CHF based on the charging session between NF and V-CHF. For example, for SMF, the V-CHF would establish the charging session with the H-CHF on a per PDU session basis.

The message flow is the same as the figure 7.2.4.1.3-1 in solution #2.1.

End of change