3GPP TSG SA WG5 Meeting #148-e 	S5-233423rev1

Electronic, 17-25 April 2023

- Source:	Huawei

- Title:	Add Solution #2.y for charging message exchange between V-CHF and H-CHF based on quota management

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed solution in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

To provide an alternative solution for handling the charging message exchange between V-CHF and H-CHF.

# 4	Detailed proposal

First change

#### 7.2.4.y	Solution #2.y: Charging message exchange between V-CHF and H-CHF based on quota management

##### 7.2.4.y.1	General

A possible solution for key issue #2e, the trigger for interaction for V-CHF communicating with H-CHF, i.e. message exchange between V-CHF and H-CHF, is based on the quota management. In specific, V-CHF will manage a bundle of quota to support the usage by NF. A bundle of quota refers to a large number of units granted by H-CHF to V-CHF for supporting a series of quota requests for a roaming subscriber, where the quota is per UE per PDU session per rating groupThe management of a bundle of quota at V-CHF could either triggered by H-CHF or V-CHF.

##### 7.2.4.y.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.y.3	Message flows

Figure 7.1.4.x.3-1 shows the message flow of charging message exchanges between V-CHF and H-CHF based on quota management for Session Based Charging (SCUR). V-CHF interacts with H-CHF when granted quota is not enough to support the request from NF. As noted, there are two possible ways of enabling the management of a bundle of quota by V-CHF, either triggered by V-CHF or H-CHF. This figure assumes that V-CHF triggers the request for a bundle of quota from H-CHF. The alternative way where H-CHF actively grant a bundle of quota to V-CHF is also applicable. The interaction between V-CHF and H-CHF for requesting and granting a bundle of quota reuses the Nchf_ConvergedCharing service API.

Step 1 to 12 describe the message flow of the delivery procedure for establishing a PDU session. At the establishment of the PDU session, V-CHF setup an interaction session with H-CHF for exchanging charging information. In addition, V-CHF request for a bundle of quota from H-CHF that are expected to support multiple requests from NF. Step 13 starts to deliver service.

- Request for service delivery: A request for session establishment is received in NF (CTF).

- Units determination: NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

- Charging data request [Initial, Quota requested]: NF (CTF) sends an initial charging data request to V-CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item b.

- Determine H-CHF: V-CHF creates a charging resource for the NF (CTF), determines which H-CHF it should contact, and generates an initial charging home request to H-CHF.

- Charging home request [Initial, Quota requested (bundled)]: V-CHF sends the request to H-CHF for the service to be granted authorization to start, and to reserve a large bundle of units that might support ongoing services.

- Account, Rating, Reservation control: H-CHF rates the charging home requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, H-CHF performs the corresponding reservations.

- Open CDR: based on policies, H-CHF opens a CDR related to the service.

- Charging home response [Initial, Quota granted (bundled)]: H-CHF grants authorization to V-CHF for the service to start, with the requested large bundle of units for V-CHF to supervise.

- Bundled quota management: V-CHF checks if the units requested by NF is sufficient based on the available number of units that has been granted by H-CHF. If the account has sufficient funds, V-CHF performs the corresponding unit reservations.

- Open CDR: based on policies, V-CHF opens a CDR related to the service.

- Charging data response [Initial, Quota granted]: V-CHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

- Granted units supervision: NF (CTF) monitors the consumption of the granted units.

- Start of service delivery: NF (CTF) starts to deliver the content/service based on the reserved number of units.

Step 14 to 19 describe the message flow of a quota management and usage reporting procedure where V-CHF has enough units reserved by H-CHF to support the current request. Step 20 describes ongoing services that could be supported by the granted bundle of quota.

- Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

- Charging data request [Update, Unit used, Quota requested]: NF (CTF) sends an intermediate charging data request to V-CHF, for more units to be granted for the service to continue and reporting the used units.

- Bundled quota management: V-CHF checks if the units requested by NF is sufficient based on the available number of units that has been granted by H-CHF. If the account has sufficient funds, V-CHF performs the corresponding unit reservations.

- Update CDR: based on policies, H-CHF updates the CDR with charging data related to the service.

- Charging data response [Update, Quota granted]: V-CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

- Granted units supervision: NF (CTF) monitors the consumption of the granted units.

- Service delivery ongoing: ongoing service delivery for additional requests.

Step 21 to 33 describe the message flow of a quota management and usage reporting procedure where V-CHF does not have enough units reserved by H-CHF to support the current request. Step 33 describes ongoing services. Step 34 release the service, followed with the post delivery procedure.

- Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

- Charging data request [Update, Unit used, Quota requested]: NF (CTF) sends an intermediate charging data request to V-CHF, for more units to be granted for the service to continue and reporting the used units.

- Bundled quota management: V-CHF checks if the units requested by NF is sufficient based on the available number of units that has been granted by H-CHF. If the account has sufficient funds, V-CHF performs the corresponding unit reservations. Otherwise, V-CHF initiates to request for additional bundle of quota from H-CHF.

- Bundled quota request trigger: A trigger associated to bundled quota management is met. Bundled unit determination is performed when applicable.

- Charging home request [Update, Unit used, Quota requested (bundled)]: V-CHF sends an intermediate charging home request to H-CHF, for additional large bundle of units to be granted for the service to continue and reporting the used units.

- Account, Rating, Reservation control: H-CHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

- Update CDR: based on policies, H-CHF updates the CDR with charging data related to the service.

- Charging home response [Update, Quota granted (bundled)]: H-CHF grants quota to V-CHF for the service to continue, with the reserved large bundle of units for V-CHF to supervise.

- Bundled quota management: V-CHF checks if the units requested by NF is sufficient based on the available number of units that has been granted by H-CHF. If the account has sufficient funds, V-CHF performs the corresponding unit reservations.

- Update CDR: based on policies, V-CHF updates the CDR with charging data related to the service.

- Charging data response [Update, Quota granted]: V-CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

- Granted units supervision: NF (CTF) monitors the consumption of the granted units.

- Service delivery ongoing: describes ongoing service delivery, which might contain additional requests for quota.

- Service release: release the service.

End of change