3GPP TSG SA WG5 Meeting #148-e 	S5-233424rev1

Electronic, 17-25 April 2023

- Source:	Huawei

- Title:	Update evaluation for key issue #1e in clause 7.2

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

To update evaluation for key issue #1e in clause 7.2 by comparing solutions for dealing with triggers for interaction between V-CHF and H-CHF, including two options for establishing interaction session and two options for triggering message exchange between home and visited CHF.

# 4	Detailed proposal

First change

### 7.2.5	Evaluation

Solutions #2.1 and #2.2 both solves key issues #2a and #2b

-	Solution #2.1 uses a new CHF-to-CHF communication, requires CHF enhancements, and solves more key issues. Allowing the CHF to also be a consumer of the Nchf_ConvergedCharging service would allow for more integration possibilities.

-	Solution #2.2 uses V-SMF to both V-CHF and H-CHF communication and is supported in the scope of 5G connectivity charging.

Solutions #2.8 and 2.9 both solves key issues #2c.

-	Solution #2.8 uses NRF to find the H-CHF.

-	Solution #2.9 uses SUPI to find the H-CHF and can supported the case when there is limited H-NRF access.

Both can be supported at the same time depending on the visitor MNO and agreement with home MNO.

Solutions #2.6 and #2.7 both solves key issues #2d.

-	Solution #2.6 reuses the Nchf_ConvergedCharging between CHFs and may add additional charging information to better support CHF-to-CHF communication.

-	Solution #2.7 uses a new Nchf service and would have different messages as well as triggers.

The H-CHF will still do converged charging for the UE in the current scenario, meaning that a new service isn’t required.

Solutions #2.10, #2.11, #2.x, #2.y and #2.12 all solves key issues #2e.

-	Solution #2.x proposes to establish charging session between V-CHF and H-CHF for each PDU session. The granularity of charging session aligns with the session between NF(CTF) and V-CHF. It is thus a simple solution.

-	Solution #2.10 proposes to establish a charging session between V-CHF and H-CHF for each UE (i.e. identified by SUPI). The granularity of charging session might not align with the session between NF(CTF) and V-CHF. This would reduce the the resource & latency in establishing and terminating sessions between home and visited CHF. However, the functionality in CHF is complex for charging session management per UE.

-	Solution #2.11 triggers the CHF-to-CHF communication based on the reception of a charging data request, this will mean that from the H-CHF point of view the triggers will happen like it would be directly from the V-SMF. The V-CHF would forward some sessions to the H-CHF.

-	Solution #2.y proposes that CHF-to-CHF communication is based on quota management. This implies that V-CHF would perform block quota management and only triggers request to H-CHF when block quota is not enough to support the request from NF. This solution would reduce V-CHF-to-H-CHF message exchange and quota management latency.

-	Solution #2.12 handles how the roaming charging profile could be used however it does not cover the CHF-to-CHF scenario.

Solution #2.10 and #2.x provide different granularity for establishing charging session between V-CHF and H-CHF. Solution #2.11 and #2.y provide different ways to handle the message exchange between V-CHF and H-CHF.

Solutions #2.1, #2.4, #2.16 all solves key issues #2f and #2g.

-	Solution #2.1 uses a new CHF-to-CHF communication, requires CHF enhancements, and solves more key issues. Allowing the CHF to also be a consumer of the Nchf_ConvergedCharging service would allow for more integration possibilities.

-	Solution #2.4 uses a new SMSF to both V-CHF and H-CHF communication.

-	Solution #2.16 uses Diameter communication between SMS-SC and charging system and is supported in the scope of SMS charging.

Solutions #2.1 and #2.5 both solves key issues #2h and #2i.

-	Solution #2.1 uses a new CHF-to-CHF communication, requires CHF enhancements, and solves more key issues. Allowing the CHF to also be a consumer of the Nchf_ConvergedCharging service would allow for more integration possibilities.

-	Solution #2.5 uses AMF to both V-CHF and H-CHF communication and is supported in the scope of 5G connection and mobility charging as is.

Editor’s note:	Further evaluations are FFS.

End of change