3GPP TSG-SA5 Meeting #148e 	S5-233432rev1

e-meeting, 17-25 April 2023

- Source:	Huawei Technologies

- Title:	pCR TR 28.839 Addition of the solution for the 5GS bridge management

- Document for:	Approval

Agenda Item:	7.5.3

# 1	Discussion

This is a pCR to TR 28.839 about addition of the solution for the 5GS bridge management.

# 2	References

[1]	3GPP TR 28.839 Study on time sensitive networking charging

# 3	Rationale

This pCR proposes to add the solution for the 5GS bridge management.

# 4	Detailed propsosal

Propose to incorporate the following change into the TR 28.839 [1].

First changes

## 7.X		Solution #X: 5GS Bridge management and configuration charging via SMF

#### 7.X.1	General

The solution #X, addresses the Key Issue #4a and #4b, covering the REQ-CH_BC-01.

Based on the clause 4.3.2 and clause 4.3.3 Non-roaming and Roaming with Local Breakout for UE requested PDU Session Establishment and PDU modification, specified in the TS 23.502 [4], the SMF supports 5GS Bridge management and configuation.

#### 7.X.2	Architecture description

Figure 7.x.2-1: SMF converged charging architecture

#### 7.X.3	Flow description

The figure 7.X.3-1 describes the high-level charging procedure for 5GS bridge information reporting based on SMF during the PDU session establishment.

Figure 7.X.3-1: 5GS Bridge management and configuration charging via SMF during the PDU session establishment

1-9.	PDU session establishment charging, based on figure 5.2.2.2.2-1 TS 32.255 [X] description. During SM Policy Association, the PCF may provide policy control request trigger for 5GS Bridge Information as defined in clause 6.1.3.5 of TS 23.503 [X]. The SMF may perform a Session Management Policy Modification procedure to report some event to the PCF that has previously subscribed.

9ch-a. The SMF sends Charging Data Request [Initial] to CHF for start of the PDU session.

9ch-b. The CHF open a CDR for this PDU session.

9ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the SMF.The CHF may provide charging related trigger for 5GS Bridge Information reporting. e.g. enable/disable the trigger or change the category.

10-20.	PDU session establishment, figure 4.3.2.2.1.1 TS 23.502 [202] description.

20ch-a. When the trigger for 5GS Bridge information available is armed, the SMF sends Charging Data Request [Update] to CHF, in order to support the integrate with IEEE TSN, if the 5GS Bridget information available is immediate report, with 5GS Bridge information (e.g. 5GS user-plane Node ID, port number of the DS-TT port, MAC address of the DS-TT Ethernet port for Ethernet PDU Session type, UE IP address for IP PDU Session type and UE-DS-TT Residence Time (if available) as provided by the UE).

20ch-b. The CHF updates the CDR for 5GS Bridget information avaiable.

20ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

The figure 7.X.3-2 describes the high-level charging procedure for 5GS bridge management and configuration charging based on SMF during the PDU session modification.

Figure 7.x-1: 5GS Bridge management and configuration charging during the PDU session modification

- PDU session establishment as described in the TS 23.502[X] clause 4.3.2.2 UE Requested PDU Session Establishment.

- SMF establish the charging session for start of the PDU session.

- SMF receive the configuration of 5GS bridge from CNC.

- The PDU session may be modified because the configuration request from CNC.

4ch-a. The SMF sends Charging Data Request [Update] to CHF for PDU session modification, with the trigger QoS change.

4ch-b. The CHF update a CDR for the configuration request from CNC.

4ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

- SMF reports the 5GS bridge information to the CNC.

5ch-a. The SMF sends Charging Data Request [Update] to CHF for the reporting of the configuration.

5ch-b. The CHF updates a CDR.

5ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

End of changes