3GPP TSG-SA5 Meeting #148e 	S5-233434rev1

e-meeting, 17-25 April 2023

- Source:	Huawei Technologies

- Title:	pCR TR 28.839 Addition of the solution for Enablers for TSC and TS Charging

- Document for:	Approval

Agenda Item:	7.5.3

# 1	Discussion

This is a pCR to TR 28.839 about addition of the solution for Enablers for TSC and TS Charging.

# 2	References

[1]	3GPP TR 28.839 Study on time sensitive networking charging

# 3	Rationale

This pCR proposes to add the solution for Enablers for TSC and TS Charging.

# 4	Detailed propsosal

Propose to incorporate the following change into the TR 28.839 [1].

First changes

#### 5.2.2.1	Use Case #2.1 Enablers for Time Sensitive Communications and Time Synchronization

If the AF is considered to be trusted by the operator, the AF could interact directly with TSCTSF, using the same service as the NEF for Time Sensitive Communications and Time Synchronization. The TSCTSF (directly or via NEF) exposes the 5GS and/or UE availability and capabilities for synchronization service to the AF as described in clause 4.15.9.2 of TS 23.502 [4].

For supporting time synchronization service requested by TS-SC, the 5GS is configured to operate in one or multiple PTP instances, including the support for derivation of Uu time synchronization error budget.

For supporting transmission gate schedules requested by TS-SC, the 5GS Bridge is required to provide Bridge Delays for each port pair and traffic class of the 5GS bridge to an TSN system

For supporting the TSC traffic characteristics requested by TS-SC, TSC QoS Flows use a Delay-critical GBR resource type and TSC Assistance Information.

The MNO may charge the TS-SC based on the enabler event for Time Synchronization Service.

The corresponding potential charging requirements and key issues: REQ-CH_ETS-01, and the Key Issue #2a, Key Issue #2b.

Next changes

## 7.X		Solution #X: Enablers for TSC and TS Charging

#### 7.X.1	General

The solution #X, addresses the Key Issue #2a and #2b, covering the REQ-CH_ETS-01.

Based on the clause 5.2.27 TSCTSF Services in TS 23.502 [4], TSCTSF Services and Service Operations are present in the following table 7.x-1.

The NF consumer AF invockes the TSCTSF service directly or via NEF based on the whether the AF is considered to be trusted by the operator or not.

Table 7.x-1: NF Services provided by the TSCTSF

| Service Name | Operation Semantics | Example Consumer(s) |
| --- | --- | --- |
| Ntsctsf_TimeSynchronization | Request/Response Subscribe/Notify | AF, NEF |
| Ntsctsf_ASTI | Request/Response | AF, NEF |
| Ntsctsf_QoSandTSCAssistance | Request/Response Subscribe/Notify | AF, NEF |

For the Time Synchronization service charging, if the NEF charging described in the clause 7.1 solution #1 is supported, TSCTSF(CTF) charging is not required. If the AF is trusted party, interactiong with TSCTSF without NEF, the TSCTSF(CTF) charging may be supported.

#### 7.X.2	Architecture description

Figure 6.1.4.2-1: TSCTSF converged charging architecture - CEF

#### 7.X.3	Flow description

#### 7.x.3.1	TSCTSF directly interact with the AF

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_TimeSynchronization message directly to the AF. The figure 7.X.3.1-1 describes the high-level charging procedure for enablers for TSC and TS Charging based on TSCTSF.

Figure 7.x.3.1-1: 5GS Bridge management charging via SMF

1.	TSCTSF receives a Time Synchronization Service Request/subscribe message from an AF, including a list of UE identities (GPSIs) or Groups of UEs identified by an External Group Identifier that further define the subset of the target UEs and AF-Service-Identifier, or DNN and optionally S-NSSAI which used to indicate the TS-SC.

2.	TSCTSF performs the actions needed to fulfil the Time Synchronization Service Request/subscribe and If authorized, the TSCTSF sends the Time Synchronization Service Response/notify message.

2ch-a. The TSCTSF sends Charging Data Request [Event] to CHF for the received Time Synchronization Service Request/subscribe.

2ch-b. The CHF creates a CDR for this Time Synchronization Service Request/subscribe.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the TSCTSF.

End of changes