3GPP TSG-SA5 Meeting #148e 	S5-233436rev1

e-meeting, 17-25 April 2023

- Source:	Huawei Technologies

- Title:	pCR TR 28.839 Addition of the solution for the TSC from TSN station to TSN system

- Document for:	Approval

Agenda Item:	7.5.3

# 1	Discussion

This is a pCR to TR 28.839 about addition of the solution for the TSC from TSN station to TSN system.

# 2	References

[1]	3GPP TR 28.839 Study on time sensitive networking charging

# 3	Rationale

This pCR proposes to add the solution for the TSC from TSN station to TSN system.

# 4	Detailed propsosal

Propose to incorporate the following change into the TR 28.839 [1].

First changes

## 7.X		Solution #X: Time Sensitive Communications charging

#### 7.X.1	General

The solution #X, addresses the Key Issue #3a and #3b, covering the REQ-CH_TSC-01, REQ-CH_TSC-02 and REQ-CH_TSC-03.

Based on the clause 5.28.1 in the TS 23.501[X], 5.28.15GS bridge management, there is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single 5GS bridge.

For time sensitive communciations, the following options are present in the figure 7.X.1:

-	TSN Bridge/End station to the TSN System: based on the PDU session 1A, the UE1-DS_TT and TSN system transfers the TSN stream via the UPF-A/NW-TT on the 5GS Bridge A.

Figure 7.x.1-1: TSC for TSN Station to TSN system

#### 7.X.2	Architecture description

Figure 7.x.2-1: SMF converged charging architecture

#### 7.X.3	Flow description

The traffic data of Time sensitive communication is transferred on the 5GS Bridge. The granularity of the 5GS TSN bridge is per UPF for each network instance or DNN/S-NSSAI. The bridge ID of the 5GS TSN bridge is bound to the UPF ID of the UPF.

#### 7.X.3.1	TSN Bridge/End station to the TSN System of the Time Sensitive Communication

The figure 7.X.3.1-1 describes the high-level charging procedure for TSN Bridge/End station to the TSN System of the Time Sensitive Communication based on SMF during the PDU session.

Figure 7.x.3-1: TSC for TSN Bridge/End station to TSN System

1:	SMF collects the usage of the TSN stream and reports the charging data information per UE per PDU session from UPF.

2ch-a.	The SMF sends Charging Data Request [Initial, Requested Quota] to the UE CHF for the TSN stream deliverly with the 5GS Bridge id and related 5GS bridge information.

2ch-b. The UE CHF creates a CDR.

2ch-c. The UE CHF acknowledges by sending Charging Data Response [Initial, Requested Quota] to the SMF.

3ch-a. The UE CCS sends Charging Data Request [Initial] to TSN CHF for the request for the TSC on the 5GS bridge.

3ch-b. The TSN CHF creates a CDR for the request for TSC.

3ch-c. The TSN CHF acknowledges by sending Charging Data Response [Initial] to the UE CHF.

For Time Sensitive Communications service, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [X].

The UE CCS charges the volume usage per UE based on the charging information reporting for the UE.

TSN CCS or billing domain aggregates the volume usage per 5GS bridge via 5GS bridge Id for TSC based on the accumulation of usage reporting per UE via the 5GS bridge.

TSN CCS or billing domain aggregates the volume usage per TSN via UPF ID for TSC based on the accumulation of usage reporting per UE via the 5GS bridge of TSN.

Editor’s note: The quota management for TSN charging (e.g. B2B) is ffs.

End of changes