3GPP TSG-SA5 Meeting #148e 	S5-233399rev1

Electronic meeting, Online, 17 -25 April 2023

- Source:	Huawei

- Title:	pCR TR 28.908 Add use case and solution for AI/ML management of RAN domain centrialized ES

- Document for:	Approval

Agenda Item:	6.7.5.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 		3GPP TR 28.908 Study on Artificial Intelligence / Machine Learning (AI/ML) management

# 3	Rationale

In TR 28.908 [1], the deployment scenarios described that the ML training function and AI/ML inference function for RAN Domain ES can be located in Domain-centralized ES function. This contribution proposes to add use case to support AI/ML management for RAN domain centralized ES function.

# 4	Detailed proposal

It is proposed to update the following chapter in TR 28.908 [1].

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.105: " Artificial Intelligence / Machine Learning (AI/ML) management ".

[5]	IBM Watson Studio, “Model Drift” [Online].

NOTE:	AVailable at: https://www.ibm.com/cloud/watson-studio/drift.

[6]	3GPP TR 28.864: "Study on Enhancement of the management aspects related to NetWork Data Analytics Functions (NWDAF)".

[7]	3GPP TR 28.310: "Management and orchestration; Energy efficiency of 5G".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[10] 	European Commission (21.04.2021): “Proposal for a Regulation laying down harmonized rules on artificial intelligence”.

[11]	High-level Expert Group on Artificial Intelligence setup by the European Commission (08.04.2019): "Ethical Guidelines for Trustworthy AI".

[12]	ISO/IEC TR 24028:2020, Information technology - Artificial intelligence - Overview of trustworthiness in artificial intelligence.

[13]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[15]	3GPP TR 37.817: "Study on enhancement for data collection for NR and ENDC".

[16]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[17]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[18]	3GPP TS 28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G".

[19]	3GPP TS 38.401: "NG-RAN; Architecture description".

[X]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[Y]	3GPP TS 28.813: "Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G ".

2nd modified section

## 5.X  Management capabilities for AI/ML Intelligence function

### 5.X.1	AI/ML management for RAN domain centralized ES function

#### 5.X.1.1	Description

Energy saving is a critical issue for the 5G operators. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NF, and the energy saving policies set by operators as specified in TS 28.310 [X] and the centralized energy saving for RAN domain area as described in TR 28.813 [Y].

By introducing AI/ML technology, 3GPP management system can provide ML training and AI/ML inference capability enabler RAN domain centralized ES function as described in TR 28.813 [Y].

#### 5.X.1.2	Use cases

With AI/ML management, 3GPP management system can query AI/ML training and inference related information of RAN domain centralized ES function e.g query AI/ML training model performance, and report ML training result or AI/ML inference result for RAN domain centralized ES function. Depending on the AI/ML management, some actions can be taken to provide AI/ML centralized ES for RAN domain area with improvements while keeping higher energy efficiency.

#### 5.X.1.3	Potential requirements

REQ-AIML_ES-1: The 3GPP management system  should have a capability to allow an authorized MnS consumer to query ML training information and AI/ML inference information for RAN domain centralized ES function.

REQ-AIML_ES-2: The 3GPP management system should have a capability to inform an authorized MnS consumer about the ML training and AI/ML inference information for RAN domain centralized ES function.

#### 5.X.1.4	Possible solutions

Extend the attribute inferencetype in the MLEntity <<dataType>> with new value e.g. inference type for RAN domain centralized ES function, which indicates the RAN domain centralized ES function type of inference that the ML model supports.

Extend the attribute inferenceOutputName in the MLEntity <<dataType>> with new value e.g. inference output name for RAN domain centralized ES function, which indicates the RAN domain centralized ES function type of inference that the ML model supports.

To support the ML training and AI/ML inference result for RAN domain centralized ES function querying and reporting, a new optional attribute can be defined in the MLTrainingReport IOC and MLInferenceReport IOC to indicate the ML training result and AI/ML inference result for RAN domain centralized ES function. The result can be including:

-    ML training information: ML entity identifier, ML entity version, ML model performance, ML training process status.

-    AI/ML inference information: ML entity identifier, ML entity version, AI/ML inference output, AI/ML infrerence process status.

MnS consumer can use “getMOIAttributes” operation to query and obtain the MnS producer about the ML training and AI/ML inference information of RAN domain centralized ES function.

End of modified section