3GPP TSG-SA5 Meeting #148e 	S5-232508rev1

Electronic meeting, Online, 17 -25 April 2023

- Source:	China Unicom

- Title:	Modify description, solution and add conclusion on issue #3

- Document for:	Approval

Agenda Item:	6.8.5

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.835 v0.6.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

# 3	Rationale

This contribution proposes to add conclusion and recommendation for Management requirement between MOP-NM and MOP-SR-DM.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

## 5.3	Issue #3:  Management requirement between MOP-NM and MOP-SR-DM

### 5.3.1	Description

For MOCN Networking Sharing, different POP needs different data for personalized operation and maintenance services (e.g. mobility management measurements focused by POP A, packet delay measurements focused by POP B). MOP-NM needs to sent different data to POP-NM for POPs’ network operation.

Figure 5.3-1: Data flow of configuration and performance measurements for NG-RAN MOCN network sharing scenario

The data flow of configuration and performance measurements is depicted in figure 5.3-1. Preventing data modification by MOP-NM and ensuring data fairness per POP, different configuration and performance measurements for each POP on MOP-SR-DM need to be forwarded to MOP-NM. Then the MOP-NM sent these data to POP-NM directly.

The data flow of alarm data is depicted in figure 5.3-2. Ensuring alarm ID continuity, different alarm data for each POP on MOP-NM need to be forwarded to each POP.

Figure 5.3-2: Data flow of alarm data for NG-RAN MOCN network sharing scenario

5.3.2	Potential solution

- The use cases of configuration, performance measurements for NG-RAN MOCN network sharing scenario can be added in TS 32.130[4].

- The use case of alarm data for NG-RAN MOCN network sharing scenario can be added in TS 32.130[4].

- Individual PerfMetricJob instance can be created and configured for each POP.

- The following attribute pLMNId can be added to PerfMetricJob IOC defined in TS 28.622[6] and used to identify the POP.

The optional attribute pLMNId can be used to establish operator-instance of PerfMetricJob IOC. The attribute performanceMetrics and optional attribute pLMNId together define the performance metrics to be produced according to individual POP’s requirements for MOCN scenario.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | CO | T | T | F | T |

The attribute constraint of pLMNId shall be added as follows:

| Name | Definition |
| --- | --- |
| pLMNId | This attribute should be supported, when the MnS producer supports the PLMN specific jobs for MOCN scenario. |

The definition of pLMNId can see in the following table.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| pLMNId | This parameter defines the information of a PLMN identification and is used to distinguish operator granularity. It is defined in TS 28.658[7]. allowedValues: N/A | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

- The getMOIAttribute operation defined in TS 28.532[8] shall have capability to filter operator-specific managed objects.

The optional parameter "filter" shall support operator-instance filtering. The pLMNId can be one filter criteria, which is applied to the objects in the sub-tree of baseObjectInstance.

- The getAlarmList operation defined in TS 28.532[8] shall have capability to filter operator-specific alarms.

The optional parameter "filter" shall support operator-instance filtering. The pLMNId can be one filter criteria, which is applied to the objects selected by the "baseObjectInstance" parameter. MnS producer shall apply pLMNId on AlarmInformation instances in AlarmList when constructing its output parameter AlarmInformationList.

2nd Change

- 6.3	Issue #3:   Management requirement between MOP-NM and MOP-SR-DM

The study has identified management requirements and solutions for personalized operation and maintenance services between MOP-NM and MOP-SR-DM. MOP-NM needs to sent different data to POP-NM for POPs’ network operation.

It is recommended to start further normative work focusing on the following main aspects in TS 32.130[4].

- 	Adding use cases of data flows for configuration, performance measurements and alarm data for NG-RAN MOCN network sharing scenario in TS 32.130.

The attribute pLMNId of PerfMetricJob IOC in TS 28.622[6], and operator-instance filtering for configuration and alarm data in TS 28.532[8] need further study and no normative work.

End of Changes