3GPP TSG-SA5 Meeting #148e 	S5-233509rev1

Electronic meeting, Online, 17 -25 April 2023

- Source:	China Unicom

- Title:	Add conclusion and recommendation for issue #2

- Document for:	Approval

Agenda Item:	6.8.5

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.835 v0.6.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

# 3	Rationale

This contribution proposes to add conclusion and recommendation for Performance measurements without PLMN ID at NRCellCU.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

- 6.2	Issue #2:   Performance measurements without PLMN ID at NRCellCU

The study has identified the issue and solutions for NRCellCU to collect performance measurements without PLMN ID.. There are two solutions to address key issue#2 in the present TR. These solutions propose the principle that the new or existing IOCs are able to support the collection of these common and operator specific measurements.

-	Solution 1 proposes that new IOCs (i.e. OperatorCU, NROperatorCellCU) could be added to the NR NRM to suport NG-RAN sharing MOCN network sharing.  The OperatorCU and NROperatorCellCU to represent the operator specific part can collect PLMN related  measurements. The NRCellCU to represent the common part of the CU can collect these common measurements (i.e. RRC.ConnEstabAtt and RRC.ReEstabAtt ).

-	Solution 2 proposes that NRCellDU is added as the measurement object class to the attempt and failure measurements of RRC connection related measurements.

The comparison of solutions is shown in the following table.

Table 6.2-1: Comparison of solutions of Issue#2

| Dimensions | Solution 1 | Solution 2 |
| --- | --- | --- |
| Measurement object impact | No change. The object of these measurements is still NRCellCU. | NRCellDU. The measurement object is NRCellDU, and the statistical point (such as RRC signaling) is in CU. The protocol layers of the two do not match. |
| Compatibility evaluation | The solution can meet potential new common performance measurement requirements of the NRCellCU. | Only resolve issues with the attempt and failure measurements of RRC connection related measurements. The solution cannot meet potential new common performance measurement requirements of the NRCellCU. |
| The impacts of technical specifications | TS 28.541 Higher impact on the NR NRM to support NG-RAN sharing MOCN network sharing. | TS 28.552 Higher impact on the measurement object class of existing performance measurements. |
| ROI | Complex for collecting non-PLMN measurements, causing big migration of configurations | Simple solution for collecting non-PLMN measurements |

For key issue #2, it need further study and no normative work.

End of Change