3GPP TSG-SA5 Meeting #149 	S5-233868

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 solution for ML Testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of Testing ML entities has been discussed and agreed (in TR 28.908). This CR is to normatively propose the solution for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | stage 2 solution has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the use case “ML Testing” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 7.X	Information model for MLTesting

## 7.X.1	Class diagram

##### 7.X.1.1	Relationship

@startuml TS 28.541 figure 6.2.1-2 (as of MArch 2021)

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class MLTesting  <<InformationObjectClass>>

class MLTestingJob << InformationObjectClass >>

class mLEntity << InformationObjectClass >>

class MLTestingRequest << InformationObjectClass >>

class MLTestingReport<<dataType>>

ManagedEntity "1" *-- "*" MLTesting

MLTesting "1" -r-> "*" MLTestingJob

MLTesting "1" *-- "*" MLTestingRequest

MLTesting "1" --> "*" mLEntity

MLTestingRequest "1" -right-> "1" mLEntity

MLTestingJob "1" --> "1"  mLEntity

MLTesting "1" *-- "*" MLTestingReport

MLTestingJob "1" --> "1"  MLTestingRequest

note left of ManagedEntity

Represents the following IOCs:

Subnetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

Figure 7.X.1.1-1: NRM fragment for ML Testing

##### 7.X.1.2	Inheritance

@startuml TS 28.541 figure 6.2.1-2

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top<<dataType>>

class ManagedFunction <<InformationObjectClass>>

class MLTesting << InformationObjectClass >>

class MLTestingRequest << InformationObjectClass >>

ManagedFunction <|-- MLTesting

Top <|-- MLTestingRequest

@enduml

Figure 7.X.1.2-1: ML Testing Inheritance Relations

## 7.X.2	Class definitions

##### 7.X.2.1 	MLTesting <<IOC>>

###### 7.X.2.1.1	Definition

This IOC represents the properties of MLTesting.

MLTesting is a managed object instantiable from the MLTesting information object class and name-contained in either a Subnetwork, a ManagedFunction or a ManagementFunction. The MLTesting is a type of managedFunction, i.e., the MLTesting is a subclass of and inherits the capabilities of a managedFunction

The MLTesting shall be associated with one or more mLEntities.

- E.g., the mLEntities associated with MLTesting may be associated via a list of mLEntityIdentifers. The mLEntityIdentifers should specify the model and any specific version thereof e.g., as defined in NC322249

- The MLTesting shall contain one or more MLTestingRequests as well as one or more MLTestingJobs , where a MLTestingJob is instantiated following or in response to one received MLTestingRequests

- MLTesting should be associated with MLTestingReport. Moreover,   MLTestingReport is name-contained within MLTesting since MLTesting shall send the reports related to the testing.

7.X.2.1.2	Attributes

The MLTesting IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLEntities | CM | T | F | F | F |
| MLTestingJobs | M | T | F | F | F |
| MLTestingRequest | M | T | T | F | T |
| MLTestingReport | M |  |  |  |  |

##### 7.X.2.2	MLTestingJob <<IOC>>

###### 7.X.2.2.1	Definition

This IOC represents the properties of MLTestingJob.

For each mLEntity under Testing, a MLTestingJob is instantiated, i.e., the MLTestingJob is associated with exactly one mLEntity. Moreover, The MLTestingJob is associated with one MLTestingRequest.

- MLTestingJob can be instantiated directly by an authorized consumer. Alternatively,  MLTestingJob can be instantiated by MLTesting in response to a MLTestingRequest.

- The MLTestingJob needs to be managed separately from MLTestingRequest, e.g., the MLTestingJob maybe instantiated directly without the existence of a related MLTestingRequest.

- Each MLTestingJob needs to be managed differently from the related mLEntity, although the MLTestingJob may be associated to only one mLEntity. The MLTestingJob may be triggered to validate a mLEntity that the MLTesting does not necessarily manage, i.e., the model may be managed by a different entity but the  MLTestingJobs may still need to be associated with that model that it is validating.

- Each MLTestingJob may have a status attribute used to indicate the level of success of the MLTestingJob.

- the MLTestingJob may be associated with one or more instances of MLTestingReport.

- The MLTestingJob may have a source to identify the entity which instantiated it and which may be used to prioritize among different MLTestingJobs from different sources. The sources may for example be an enumeration defined for network functions, operator roles, or other functional differentiations.

- Each MLTestingJob will have attributes specifying either the MLTestingData to be used to validate the mLEntity (function) or an expectedRuntimeContext for which the mLEntity (function) should be validated.

- Each MLTestingJob will have attributes specifying the MLTestingReportingPeriod (e.g., periodically, after completion, etc.)

- The reporting requirements contained in the MLTestingJob must be mapped to an existing MLTestingReport instance or create a new instance as needed.

7.X.2.2.2	Attributes

The MLTestingJob IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLTestingJobID | M | T | F | F | F |
| mLEntityID | M | T | F | F | F |
| MLTestingRequest | M | T | T | F | T |
| Source | M | T | T | F | T |
| MLTestingData | O | T | T | F | T |
| expectedRuntimeContext | O | T | T | F | T |
| ProgressStatus | M | T | T | F | T |
| MLTestingReport | M | T | F | F | T |
| MLTestingReportingPeriod | O | T | T | F | T |

##### 7.X.2.3 	MLTestingRequest <<IOC>>

###### 7.X.2.3.1	Definition

This IOC represents the properties of MLTestingRequest.

For each request to undertake Testing, a consumer creates a new MLTestingRequest on the MLTesting, i.e.,  MLTestingRequest shall be an information object class that is instantiated for each request for Testing

- Each MLTestingRequest is associated to exactly one mLEntity

- Each MLTestingRequest may contain specific reporting requirements including an MLTestingReportingPeriod  that define how the MLTesting may report about the MLTestingRequest or the associated MLTestingJob.

- The MLTestingRequest may be associated with a MLTestingJob that is instantiated in response to the MLTestingRequest.

- The MLTestingRequest may have a source to identify where its coming from and which may be used to prioritize among different MLTestingRequests from different sources. The sources may for example be an enumeration defined for network functions, operator roles, or other functional differentiations.

- Each MLTestingRequest may have a RequestStatus field that is used to track the status of the specific MLTestingRequest or the associated MLTestingJob.

- Each MLTestingRequest will have attributes specifying either the MLTestingData to be used to validate the mLEntity (function) or an expectedRuntimeContext for which the mLEntity (function) should be validated.

- The RequestStatus is written by the MLTesting when there is a change in status of the Testing progress. The RequestStatus is an enumeration with the following values:

  - "Pending": Indicates that the  MLTestingRequest has been received but no action has been taken thereof. When other actions are taken regarding the MLTestingRequest, the RequestStatus changes to one of the following values.

  - "Triggered": when a corresponding MLTestingJob has been instantiated and is running

  - "Suspended": the status when for one reason or another the MLTestingRequest has been suspended by the MLTesting or another authorized consumer, e.g. the operator.

  - "Served": the status when an MLTestingJob associated  to the specific MLTestingRequest has run to completion.

- The reporting requirements contained in the MLTestingRequest must be mapped to an existing MLTestingReport instance or create a new instance as needed.

7.X.2.3.2	Attributes

The MLTestingRequest IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLTestingRequestID | M | T | F | F | F |
| mLEntity | M | T | F | F | F |
| MLTestingJob | M | T | F | F | F |
| Source | M | T | T | F | T |
| RequestStatus | M | T | T | F | T |
| MLTestingData | O | T | T | F | T |
| expectedRuntimeContext | O | T | T | F | T |
| MLTestingReportingPeriod | O | T | T | F | T |

## 7.X.3	DataType definitions

##### 7.X.3.1 	MLTestingReport << dataType >>

###### 7.X.3.1.1	Definition

This dataType represents the properties of MLTestingReport.

- MLTesting may generate one or more MLTestingReports,

- Each MLTestingReport is associated to one or more mLEntities.

- MLTestingReport provides information about mLEntities or MLTestingJobs that themselves are associated with mLEntities for which testing is requested and/or executed.

7.X.3.1.2	Attributes

The MLTestingReport includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLTestingReportID | M | T | F | F | F |
| MLTestingJobID | M | T | F | F | F |
| mLEntities | M | T | F | F | F |
| MLTestingRequests | O | T | F | F | F |

## 7.X.4	Attribute definitions

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLEntities | It indicates the list of ML models available at the MLTesting function | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLTestingJobID | It indicates an identifier for a specific instantiated MLTesting job | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLEntityID | It indicates an identifier for a specific mLEntity . It may include the version identifiers of any such mLEntity | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| Source | It indicates a managed object that initiates the request for MLTesting | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLTestingData | It indicates the data to be used for testing the model | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| expectedRuntimeContext | It describes the conditions and characteristics of the environment where the mLEntity is expected to be used for inference | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| ProgressStatus | It indicates the status of the MLTesting process as evaluated by the MLTesting function. | type: DN (see TS 32.156 [12]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLTestingReportingPeriod | It defines how the MLTesting may report about the MLTestingRequest or the associated MLTestingJob | type: DN (see TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLTestingReportID | It indicates an identifier for a specific MLTestingReport instance | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingReportID | It indicates an identifier for a specific MLTesting report | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of Changes