3GPP TSG-SA5 Meeting #149	S5-233948

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Input to draft CR TS28.105 Event data for ML training use case and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use cases of “Event data for ML training” has been discussed and agreed upon in the study (clause 5.1.1 in TR 28.908). This CR is to normatively propose the use case and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the use cases description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.M.2.z (new), 6.M.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.105 DraftCR S5-233950 |  |  |
| Other comments: |  | The caluse 6.M (6.M.1 Description and struture 6.M.2 Use Case/6.M.3 Requirements) was introduced in DraftCR S5-233950. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 6.M.2.z	Pre-processed Event data for ML training

For AI/ML algorithms, a large amount of data points does not necessarily add value, e.g., if most of it includes biased data which ends up getting discarded during the pre-processing stages. Instead, the AI/ML algorithms need to have information-rich events data that is condensed but with most of it useful for the required training. For example, one could train an interference optimization solution that learns the best way to combat interference by looking at correlated event which derived from counters of handover failures correlated with signal quality.

It is as such necessary to provide means to isolate and store the information rich network events in the network, to ensure that minimizing storage and processing costs by discarding the unnecessary raw data does not compromise the ability to and still avails adequate historical information to adequately train ML entities. In other words, it is necessary for network functions and their management system to generate data about the observed network events, e.g., based on the criteria set by the operator. The network events can then be stored to be used later to train ML entities.

Here, a network event represents a specific combination of activities executed on the network or occurrences in the network identified say by a set of instances of threshold crossings.

Figure 6.M.2.z-1. Exposing and storing network events data

As maybe seen in Figure 6.M.2.z-1, there could be multiple sources for the same event data e.g., either the source network function or a related management function. Correspondingly, it may be necessary to eliminate duplications of events or event data that is exposed to the ML inference functions. It may in such cases make sense to have a single events data exposure entity, say called a network events aggregator that exposes data from multiple sources but without duplications.

Relatedly, when an MnS consumer for ML training producer wishes to trigger a training based on such events and not on specific raw data, such a consumer should be enabled to request for such training and clarify that they have training executed based on network events.

Next Modification section

### 6.M.3	Requirements for ML training

Table 6.M.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-EVENT-DATA-1: | The 3GPP management system should enable an authorized consumer to request from the network data producer for network events corresponding to the data produced by that network data producer. | Pre-processed Event data for ML training (clause 6.M.2.z) |
| REQ-EVENT-DATA-2: | The 3GPP management system should enable a network data producer to generate network events. | Pre-processed Event data for ML training (clause 6.M.2.z) |
| REQ-EVENT-DATA-3: | The 3GPP management system should enable a network events aggregator to take the events from different network entities and re-expose them in an aggregated way that eliminates duplications. | Pre-processed Event data for ML training (clause 6.M.2.z) |
| REQ-EVENT-DATA-4: | The 3GPP management system should enable an authorized MnS consumer to configure the ML Training MnS producer to provide training based on a set of network events. | Pre-processed Event data for ML training (clause 6.M.2.z) |
| REQ-EVENT-DATA-5: | The 3GPP management system should have a capability to inform an authorized MnS consumer when training is triggered based on an MnS consumer’s request for training based on a set of network events. | Pre-processed Event data for ML training (clause 6.M.2.z) |

End of Modification