3GPP TSG-SA5 Meeting #149 	S5-234105

# Berlin, Germany, 22-26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0034 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct EAS to connect to UPF procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to clause 7.4.4 in TS 28.538 and TS 28.541, only EcmConnectionInfo IOC is introduced for connection between EDN NF and 5GC NF, including connection between EAS and UPF. Having UPFConnection IOC in figure 7.4.4-1 is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change UPFConnection IOC to EcmConnectionInfo IOC in figure 7.4.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May lead to wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

7.4.4	EAS to connect to UPF

Figure 7.4.4-1 depicts a procedure to describe how ECSP management system can consume provisioning MnS to request PLMN management system to connect EAS to UPF for transporting the user traffic via the N6 interface (see clause (see clause 4.2.3 in TS 23.501 [11]). To support the connection of EAS NF to UPF NF, EcmConnectionInfo IOC should include the following attributes:

-	EAS IP address: indicate the UPF IP address.

-	EAS and EDN service area requirements: EAS service area (see clause 7.3.3 in TS 23.558 [2]).

-	ECM connection type: indicate the user plane connection

-	N6 traffic routing list: each entry in the list should contain the following attributes (see clause 8.2.4 in TS 23.558 [2]):

-	DNAI: DNAI(s) associated with the EAS.

-	N6 traffic routing requirements: N6 traffic routing information corresponding to each EAS DNAI.

-	UPF Connection information: contains the following attributes:

-	UPF IP address: the IP address of the accessing NF.

-	UPF DN: the UPF DN to be configured in EASFunction IOC to indicate where the UPF is connected.

Figure 7.4.4-1: EAS to connect to UPF

1. ECSP management system consumes the provisioning MnS with createMOI operation (see clause 11.1.1.1. in TS 28.532 [5]) for EcmConnectionInfo IOC to request PLMN management system to connect the EAS to an UPF. EcmConnectionInfo includes EAS IP address, EAS service area , EDN service area and ecmConnectionType with value USERPLANE.

2. PLMN management system finds a UPF based on the EAS and EDN service areas.

If an UPF can be found, then performs the following steps:

3. PLMN management system creates the EcmConnectionInfo MOI with uPFConnectionInfo, including UPF IP address and UPF DN.

4. PLMN management system create EP_N6 MOI with UPF IP address in localAddress, and EAS IP address in remoteAddress to connect UPF to EAS.

5. PLMN management system returns the UPF connection information in the attributeListOut of the output parameter in createMOI operation to ECSP management system.

6. ECSP management system create EP_N6 MOI with EAS IP address in localAddress, and UPF IP address in remoteAddress.

If an UPF cannot be found, then performs the following steps:

7. PLMN management system returns the output parameters for createMOI operation to indicate the UPF instantiation is in progress.

8. PLMN management system invokes the InstantiateNsRequest operation (see clause 7.3.3 in ETSI GS NFV-IFA 013 [6]) to request NFVO via the Os-Ma-nfvo interface to instantiate a NS instance including the UPF VNF instance.

9. NFVO sends a notification to PLMN management system indicating the result of instantiation procedure (see clause 7.3.3.4 of ETSI GS NFV-IFA 013 [6]).

10. If the UPF has been instantiated, then performs the following steps:

10.1. PLMN management system creates the EcmConnectionInfo MOI with UPF connection information, including UPF IP address and UPF DN.

10.2. PLMN management system create EP_N6 MOI with UPF IP address in localAddress, and EAS IP address in remoteAddress to connect UPF to EAS.

10.3 PLMN management system sends notifyMOICreation with UPF connection information in attributeList.

10.4. ECSP management system create EP_N6 MOI with EAS IP address in localAddress, and UPF IP address in remoteAddress.

11. If the UPF has not been instantiated, then performs the following step:

11.1 PLMN management system sends notifyMOICreation to ECSP management system to indicate no UPF can be found.

End of Change