3GPP TSG-SA5 Meeting #149 S5-234433 # Berlin, Germany, 22 – 26 May, 2023 CR-Form-v12.2. CHANGE REQUEST. 28.554; CR; 0126; rev; -; Current version:; 18.1.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New KPI “NR and EUTRAN data volume retention rate” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 28.554, currently there is no KPI quantifying the relative utilization of different RATs, i.e. 5G NR and LTE, in terms of the data volume. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new KPI showing what percentage of the network PDCP PDU Data Volume are on 5G NR as compared to E-UTRAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | As 5G NR gets widely deployed, there will be no KPI indicating the adoption of the 5G NR in term of the user data volume. This KPI is crucial in identifying the adoption of a new radio access technolgy. Furthermore, it can be utilized to determine sunsetting an older radio access technologiy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.552 CR0441 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 6.5.X	RAT Retainability

#### 6.5.X.3	NR and E-UTRAN data volume retention rate

a)	DataVolumeRetentionRate_5G, DataVolumeRetentionRate_E-UTRAN.

b)	This metric shows what percentage of the network PDCP SDU Data Volume are on 5G as compared to E-UTRAN.

c) 	To measure the RRC retention rate:

DataVolumeRetentionRate_5G=Combi.DRB. PdcpSduVolumeDL.5GCombi.DRB. PdcpSduVolumeDL

DataVolumeRetentionRate_EUTRAN=Combi.DRB. PdcpSduVolumeDL.ECombi.DRB. PdcpSduVolumeDL

Measurement names: Combi.DRB. PdcpSduVolumeDL, Combi.DRB. PdcpSduVolumeDL.E, Combi.DRB. PdcpSduVolumeDL.5G

On SubNetwork level, the measurement shall be aggregated over all NRCellCUs/ EUtranCell in the SubNetwork.

d) 	SubNetwork (or at the DN of an object instance which has access to Combi metrics).

# >>>>END OF CHANGES<<<<