# 3GPP TSG-SA5 Meeting #149	S5-234538

# Berlin, Germany, May 22-26, 2023                                             	was S5-234063

# Source:	Samsung, Nokia, Deutsche Telekom

# Title:	Rel-18 pCR 28.829 7.1.2.1 Solution #1 correction

# Document for:	Approval

Agenda Item:	6.9.2.4 (FS_NSOEU_WoP#4 Capture solutions for Objective #1)

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None specific to this pCR.

# 3	Rationale

This pCR proposes a correction to the figure 7.1.2.1.2-1, for improved alignement to existing specifications. A few editorial changes are made as well, with advice from MCC EditHelp.

changes in S5-234538, revision of S5-234063

- align the terminology "Performance Assurance Producer" (one instance of "Performance Assurance MnS Producer" needed to be changed. [Huawei]

- step 4 needs clarification on 'configured reporting' (from step 1) [Telefonica]

- add support from Deutsche Telekom

- correct a typographic error [Deutsche Telekom]

# 4	Detailed proposal

It is proposed to agree to the following change to TR 28.829, 1.1.0.

Begin Change

#### 7.1.2.1	Potential Solution #1: MNO exposes network performance monitoring

##### 7.1.2.1.1	Introduction

This solution addresses Key Issue 1 and the requirements in clause C.6.

The solution assumes that DSO is an MNO trusted entity and can access MnSes provided by the 3GPP Management System. In this solution, the existing provisioning MnS and related NRM fragments are used. DSO uses ThresholdMonitor to configure the related threshold for various network performance requirements e.g. latency, throughput, packet loss, cell/network availability etc. DSO also uses the NtfSubscriptionControl to subscribe for NotifyThresholdCrossing notifications. The management system monitors the network and delivers the notification when the measurement or KPI crosses the configured threshold limit.

Depending on the configuration of the monitorGranularityPeriod attribute of the ThresholdMonitor, the DSO can identify the frequency of the threshold to be checked, post which the notification will be sent.

The returned Measurements conform to the Measurement Template [15], which identifies each individual measured value distinctly. Thus, it is possible to ascertain from the report the number of samples per measurement as well as the distribution of the data (e.g. to compute the standard deviation.)

##### 7.1.2.1.2	Description

Figure 7.1.2.1.2-1: Monitoring Configuration Procedure

1.	DSO  sends createMOI request for perfMetricJob  IOC to Performance Assurance Producer.

NOTE: The configured reporting mechanisms (file or stream) are established in this step.

2.	createMOI response is sent by Performance Assurance Producer to DSO.

3.	Performance Assurance producer creates the PM jobs on the specified object instances.

4.	Performance Assurance Producer provides the required measurements and KPIs to DSO based on the configured reporting mechanisms (file or stream).

5.	DSO sends createMOI request for ThresholdMonitor IOC to Performance Assurance Producer.

5a.	The attribute performanceMetrics,  contains the measurements defined for each of the network performance requirements as required by DSO e.g. latency, throughout, packet loss, availability etc.

5b.	It  also contains an attribute containing the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.

6.	Performance Assurance Producer  creates the MOI.

7.	createMOI response is sent by Performance Assurance Producer to DSO.

8.	DSO sends createMOI for NtfSubscriptionControl IOC to Performance Assurance Producer.

8a.	The attribute notificationRecipientAddress  contains the address of the notification recipient i.e. DSO.

9.	Performance Assurance Producer  creates the MOI.

10.	createMOI response is sent by Performance Assurance Producer to DSO.

11.	Performance Assurance Producer checks for threshold crossing based on the current state of the related    performance measurement.

12.	Performance Assurance Producer sends notifyThresholdCrossing notification to DSO if the performanceMetrics     value crosses the configured thresholdValue.

End of Changes