3GPP TSG-SA5 Meeting #151	S5-236411

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | draftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Input to draft CR TS28.104 Event data analytics use case and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use cases of “Event data for ML training” has been discussed and agreed upon in the study (clause 5.1.1 in TR 28.908). This CR is to normatively propose the use case and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the use cases description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 7.2.x	MDA capabilities for general data analytics

#### 7.2.x.1 Event data Analytics

#### 7.2.x.1.1 Description

This MDA capability is for analysis of Event data.

##### 7.2.x.1.2 Use case

Given the amount of data generated in mobile networks, it is necessary to optimize data storage by discarding older data while keeping adequate data to enable effective conducting analytics tasks (e.g., training of the ML models). However, a large amount of data points does not necessarily add value, e.g., if most of it includes biased data which ends up getting discarded during the pre-processing stages. So, the data consumer (e.g., ML model) instead need to be provided with information-rich events data that is condensed but with most of it useful for the required analytics tasks. This then ensures that minimizing storage and processing costs by discarding the unnecessary raw data does not compromise the ability to avail adequate information to adequately conducting analytics tasks (e.g., train ML models). For example, an interference optimization solution for combating interference may be trained on correlated events data which is derived from counters of handover failures correlated with signal quality.

It is as such necessary to provide means to analyse the raw data and store the information rich events in the network, to ensure that minimizing storage and processing costs by discarding the unnecessary raw data does not compromise the ability to and still avails adequate historical information to adequately for other analytics task (e.g, train AI/ML models). In other words it is necessary for network functions to their management system to perform event data analytics so as to generate data on about the observed network events, e.g., based on the criteria set by the Operator, which events can then be stored to be used later to for other tasks, e.g., train AI/ML models.

Here, a network event represents a specific combination of activities executed on the network or occurrences in the network identified say by a set of instances of threshold crossings. The network functions and the management system need to generate data on the observed network events, e.g., based on the operator's criteria.

Network Function

AI/ML enabled Function as an example of Network events Data Consumer

Network Function

Events  data analytics

Events data

Management Function

Events data analytics

Network/system data

Network/system events data

Figure 7.2.x.1.2-1. Analytics of network events data

##### 7.2.x.1.3 Requirements

Table 7.2.x.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-EVENT-DATA-1 | MDA capability for event data analytics should enable an authorized consumer to request event data analytics for network events corresponding to a certain network data | event data analytics (clause 7.2.x.1) |

End of Modification