3GPP TSG-SA5 Meeting #151	S5-236814

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0055 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and requirements for AIML inference configuration management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 6.5 provide the ML entity inference phase. Therefore, it is necessary to add the use case which has been recommended in the TR 28.908 for AIML inference configuration management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added use case and requirements for AIML inference configuration management based on TR 28.908. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case and requirements for AIML inference configuration management is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.5	AI/ML inference phase

6.5.4	AIML inference configuration management

6.5.4.1	Description

The AI/ML inference function needs to be configured (e.g., with policies, targets, conditions where applicable) in order to conduct inference in the 5G system aligning with the consumer´s expectation.

6.5.4.2	Use cases

6.5.4.2.1	Configuration for AI/ML inference initiated by MnS consumer

The MnS consumer monitors the network performance and determines on whether to, and when to trigger the AI/ML inference configuration or re-configuration. For example, for NG-RAN intelligence ES function (as described in TR 37.817 [b]), the MnS consumer collects the performance data of the capacity booster cells and coverage cells, then makes the decision for configuring or re-configuring the inference function with a policy may include e.g., performance targets of the inference function, or the activation/deactivation of the ES function and/or the associated ML entities.

In this case, the MnS consumer may need to initiate the AI/ML inference configuration/reconfiguration.

The inference function has a set of configurable attributes whose values can be changed by an MnS consumer. As such the MnS consumer may set the values of these inference function configuration attributes. Note that inference function configuration attributes are different from Network Configuration Parameters (NCPs), which are the actual network parameters whose values are set by the inference function. Changes in inference function configuration attributes values translate into changes in the behaviour of the inference function but not necessarily in the instantaneous behaviour of the network. An example inference function configuration attribute is the maximum allowed value to which a configuration parameter may be set, e.g., the maximum value to which or by which a cell individual offset may be adjusted.

6.5.4.2.2	Configuration for AI/ML inference selected by producer

The MnS producer monitors the network performance and determines on whether to, and when to trigger the AI/ML inference configuration or re-configuration. For example, NG-RAN intelligence ES function (as described in TR 37.817 [b]), per the performance of the energy efficiency result by execution of the inference output, the MnS producer may decide to activate or deactivate the inference function, or decide to use another ML entity for inference. In this case, the MnS producer may initiate the configuration and inform an authorized MnS consumer about the configurations. The configuration actions conducted by the MnS producer may also be triggered by a predefined configuration policy.

Inference functions are characterized by contexts e.g., via the associated ML entities (-cf. TS 28.105[4] and clause 5.1.7 ML context). Networks (and thus inference functions) will have several contexts which all cannot be addressed by a single configuration setting. To enable, the MnS producer to monitor the network performance and determines on whether to, and when to trigger the AI/ML inference configuration or re-configuration, the MnS producer should be configured with objectives based on which the MnS producer can configure its attributes.

6.5.4.3	Requirements for AIML inference configuration management

Table 6.4.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF_CFG-01 | The AI/ML Inference MnS producer should have a capability to allow an authorized MnS consumer to configure the inference function. | Configuration for AI/ML inference initiated by MnS consumer (clause 6.5.4.2.1) |
| REQ- AI/ML_INF_CFG-02 | The AI/ML Inference MnS producer should have a capability to configure inference function and inform an authorized MnS consumer about the configurations of the AI/ML inference function. | Configuration for AI/ML inference initiated by MnS producer (clause 6.5.4.2.2) |

End of change