# 3GPP TSG-SA5 Meeting #151	S5-236840

# Xiamen, China, October 9-13, 2023

# Source:	Nokia

# Title:	Rel-18 pCR 28.318 Annex A Procedures

# Document for:	Approval

Agenda Item:	6.6.4.1 (NSOEU_WoP#1) General

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

This pCR is motivated by reference to preceding work relevant to the feature.

[1] 3GPP TR 28.829, "Study on network and service operations for energy utilities".

# 3	Rationale

A DSO can implement a service that makes use of exposed information from the management system by accessing capabilities that are already defined in the specificationss today. This section provides informative examples of the specific steps a DSO can take to implement the desired functional service.

# 4	Detailed proposal

It is proposed to agree the following change to TS 28.318 0.1.0.

Begin Change

# Annex A (informative): Procedures

## A.1	Exposing performance and prediction information for high availability

### A.1.1	DSO is trusted and supported by network operator

#### A.1.1.1	General

In this scenario the DSO consumer, is recognized by the network operator as a trusted entity that is allowed to read from and write to selected producers and corresponding MOIs in one or more MIBs controlled by the network operator. In this basic scenario, the network operator supports the DSO consumer by providing specific necessary information such as a full Distinguished Name of producer(s) and MOI(s), by some means not described in this TS.

#### A.1.1.2	Signal flow – setup

The DSO consumer wants to receive performance metric reports from a specific cell site. The DSO consumer is in possession of the following information:

the identity of the cell site: base station ID, or

the estimated location of the cell site antenna

#### NOTE: Based upon the identity of the cell site, the location of the cell site, or both, the supporting site operator provides to the DSO the full DN and any required scoping parametes for a PerfMetricJob MOI, a ThresholdMonitor MOI and a NtfSubscriptionControl MOI.

Figure A.1.1.2-1

- PerfMetricJob

1.	The DSO consumer sends createMOI to the producer with the following parameters: managedObjectClass = PerfMetricJob, managedObjectInstance = {DN supplied by supporting site operator}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| jobId | <string, chosen by DSO> |
| performanceMetrics | DRB.AirIfDelayDL_fliter, DRB.AirIfDelayUl_filter, DRB.UEThpDL, DRB.UEThpDL.QOS, DRB.UEThpDLSNSSAI, DRB.UEThpDL.PLMN, DRB.UEThpUL, DRB.UEThpUL.QOS, DRB.UEThpULSNSSAI, DRB.UEThpUL.PLMN, DRB.PacketLossRateUL, DRB.PacketLossRateUL.QOS, DRB.PacketLossRateUL.SNSSAI and cell/network availability |
| granularityPeriod | <integer, defined in seconds, chosed by DSO> |
| objectInstances | included if supplied by the supporting site operator |
| rootObjectInstances | included if supplied by the supporting site operator |
| reportingCtrl | fileReportingPeriod = <integer, multiple of granularity period, chosen by DSO> |
| _linkToFiles | - |

In this example, report files shall be stored on MnS producer and, on condition that an appropriate subscription is in place, inform the MnS consumer about the availability of new files and the file location using the notifyFileReady notification. Other reporting configurations are possible according to attribute settings.

2.	The producer creates “PerfMetricJob” data collection job and Files MOI to contain report information.

3.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Metric production is now active on the in-scope object instances whose object class matches the object class associated to the specified performance metrics.

Repeat steps 1 - 3 for each required MOI where performance metric information is wanted.

- ThresholdMonitor

4.	The DSO consumer sends createMOI for ThresholdMonitor to monitor the requirement performance measurements including measurement for latency, packet loss, throughput and cell/network availability, using the following parameters: managedObjectClass = ThresholdMonitor, managedObjectInstance = {DN supplied by supporting site operator}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| thresholdInfoList | list of ThesholdInfo <<datatype>>, one for each performance metric. chosen by DSO |
| monitorGranularityPeriod | <integer, defined in seconds, multiple of granularity period from PMJ MOI, chosen by DSO> |
| objectInstances | included if supplied by the supporting site operator |
| rootObjectInstances | included if supplied by the supporting site operator |

In this example, a ThresholdInfo datatype will be included for each performance metric that was included in the creation of the corresponding PerfMetricJob MOI.

ThresholdInfo will also contain an attribute for the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.

5.	The producer creates “ThresholdMonitor” data collection job.

6.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 4 - 6 for each required MOI where threshold monitoring is required.

- NtfSubscriptionControl

7.	The DSO consumer sends createMOI for NtfSubscriptionControl to subscribe for the required notiifcations with the follows parameters: managedObjectClass = NtfSubscriptionControl, managedObjectInstance = {DN supplied by supporting site operator}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| notificationRecipientAddress | <string, address of the notification recipient>. |
| notificationTypes | notifyFileReady, notifyFilePreparationError, notifyThresholdCrossing |
| scope | included if supplied by the supporting site operator |
| notificationFilter | - |

The address of the notification recipient is provided by the DSO. In this example, three notification types are allowed and none are filtered.

8.	The producer creates “NtfSubscriptionControl” job.

9.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 7 - 9 for each cell required MOI where threshold monitoring is required.

A.1.1.3	Signal flow – completion

The DSO consumer wants to end performance metric reports from a specific cell site which was previously setup for performance metric reporting. For ultimate deactivation of metric production and threshold monitoring the MnS consumer should delete the job and monitor to free up resources on the MnS producer. Deletion of NtfSubscriptionControl is mandatory according to 28.622 4.3.22.1.

NOTE: The supporting network operator provides to the DSO the full DN and any required scoping parametes for a PerfMetricJob MOI, a ThresholdMonitor MOI and a NtfSubscriptionControl MOI.

Figure 1.1.3-1

- PerfMetricJob

- The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by the supporting site operator}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the supporting site operator.

- The producer deletes “PerfMetricJob” data collection job.

- The producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Best effort synchronization applies; all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it. The managed objects selected for this operation are specified by information provided by the supporting site operator.

When the "PerfMetricJob" is deleted, the ongoing reporting period shall be aborted, for streaming the ongoing granularity period.

- ThresholdMonitor

- The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by supporting site operator}, scopeType, scopeLevel and Filter are optional and SHALL be included if supplied by the supporting site operator.

- The producer deletes “ThresholdMonitor” data collection job.

- The producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Best effort synchronization applies; all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it.

- NtfSubscriptionControl

- When the DSO consumer does not wish to receive notifications any more, it shall delete the corresponding NtfSubscriptionControl instance. The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by supporting site operator}, scopeType, scopeLevel and Filter are optional and SHALL be included if supplied by the supporting site operator.

- The producer deletes “NtfSubscriptionControl” job.

- The producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

End of Changes