3GPP TSG-SA5 Meeting #151 	S5-236857

Xiamen, China, 09 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0150 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.554 Add new KPI on DL reliability with delay threshold in RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | URLLC_Mgt |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN Reliability KPIs in TS 28.554 is defined based on packet loss rate measurement in 4.2.1.5.1 in TS 38.314. However, because the existing measurement isn’t suitable for delay critical service, in the RAN2 121-bis meeting, a new measurement named “Packet Uu Loss Rate with delay threshold in the DL per DRB per UE” is introduced into TS 38.314 as a compensation to the existing packet loss rate measurement. The new measurement is used to define the packet loss rate especially for delay critical service so as to cover all the application scenario. Accordingly, a new reliability KPI with delay threshold should be added into TS 28.554 to cover delay critical service application scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new KPI for evaluating of reliability in Downlink for NG-RAN taking delay threshold into consideration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.8.1.x	Packet transmission reliability KPI with delay threshold in DL on Uu

a)	DLRelPSRDT_Uu

b)	This KPI describes the downlink reliability with delay threshold between gNB and UE. It is used to evaluate the Uu interface reliability contribution to the total network downlink reliability. It is the percentage of RLC SDU packets which have been transmitted over the air and positively acknowledged and delayed no more than the corresponding delay threshold out of the total RLC SDU packets transmitted by gNB. It is a measure of the DL packet delivery reliability over Uu interface. It is a percentage value (%). This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)	Below is the equation for downlink Reliability with delay threshold in RAN between gNB and UE.

DLRelPSRDT_Uu  =  N_dt(T,drbid)N_dtT,drbid+DlossT,drbid+DexdT, drbid   ×  100 , where 
N_dt(T,drbid), Dloss(T,drbid) and Dexd(T,drbid) are as defined in TS 38.314.

or optionally DLRelPSRDT_Uu.QoS =   N_dtT,drbid.QOSN_dtT,drbid.QOS+DlossT,drbid.QOS+DexdT, drbid.QOS   ×  100, where QoS identifies the target QoS quality of service class.

or optionally DLRelPSRDT_Uu.SNSSAI =  NdtT,drbid.SNSSAIN_dtT,drbid.SNSSAI+DlossT,drbid.SNSSAI+DexdT, drbid.SNSSAI   ×  100, where SNSSAI identifies the S-NSSAI.

d)  NRCellDU

End of Change