3GPP TSG-SA5 Meeting #151	S5-237136

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0219 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.531 Clarify initiation of Procedure of Network Slice Instance Deallocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure of Network Slice Instance Deallocation may be initated with DeallocateNsi operation or using deleteMOI operation. This is not mentioned in the description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description enhanced to clarify that Procedure of Network Slice Instance Deallocation may be initated with DeallocateNsi operation or using deleteMOI operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent/incomplete specification leads to confusion and incorrect implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-236328 is revised to S5-237136 |  |  |  |  |  |  |  |  |

1st Change

## 7.4	Procedure of Network Slice Instance Deallocation

Figure 7.4-1 depicts the procedure of deallocating a network slice instance by the network slice management service provider to satisfy the NSI deallocation request received from an authorized consumer.

Figure 7.4-1: Network slice instance deallocation procedure

1a-1) Network Slice Management Service Provider (NSMS_P) receives a DeallocateNsi request (see DeallocateNsi operation defined in clause 6.5.3) from Network Slice Management Service Consumer (NSMS_C) indicating that the NetworkSlice MOI is no longer needed for the given requirements i.e ServiceProfile.

1a-2)	NSMS_P identifies the DN of NetworkSlice MOI from the attribute networkSliceRef and value of attribute serviceProfileId of the NetworkSliceController MOI to be deleted.

1a-3)	NSMS_P updates attribute networkSliceControllerRef of NetworkSlice MOI (identified in step 1b-2) attribute to remove the DN of NetworkSliceController MOI to be deleted.

1a-4)	NSMS_P deletes NetworkSliceController MOI.

1a-5) The NSMS_P sends the DeallocateNsi response (see DeallocateNsi operation defined in clause 6.5.3) to NSMS_C.

1b-1)	NSMS_P receives a deleteMOI request (see deleteMOI operation defined in TS 28.532 [8]) from NSMS_C with network slice controller (see NetworkSliceController IOC and its attribute defined in TS 28.541[6])).

1b-2)	NSMS_P identifies the DN of NetworkSlice MOI from the attribute networkSliceRef and value of attribute serviceProfileId of the NetworkSliceController MOI to be deleted.

1b-3)	NSMS_P updates attribute networkSliceControllerRef of NetworkSlice MOI (identified in step 1b-2) attribute to remove the DN of NetworkSliceController MOI to be deleted.

1b-4)	NSMS_P deletes NetworkSliceController MOI.

1b-5)	NSMS_P sends the response to NSMS_C for the deleteMOI request.

NOTE: Network slice deallocation may be initiated using DeallocateNsi operation as described in steps 1a or using deleteMOI operation as described in steps 1b.

2) The NSMS_P may decide to delete or modify the NetworkSlice MOI (identified in step 1b-2).

3a-1)	If NSMS_P decides to delete the NetworkSlice MOI, then it deletes the NetworkSlice MOI.

3a-2)	NSMS_P invokes the procedure of network slice subnet instance deallocation as described in clause 7.5.

3b-1) If NSMS_P decides to modify the NetworkSlice MOI, procedure of network slice subnet instance modification procedure as described in clause 7.6.

3b-2)	NSMS_P modifies the NetworkSlice MOI to update the serviceProfileList (using the serviceProfileId identified in step 1a-2) or 1b-2) and may update other attributes.

If the NSMS_C has subscribed for the notification, then NSMS_P sends notification for deletion (see notifyMOIDeletion operation defined in TS 28.532 [8]) and notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceController MOI and NetworkSlice MOI.

End of change