3GPP TSG-SA5 Meeting #152	S5-237629

# Chicago, US, 13-17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0419 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.422 Addressing tracing gaps in signalling based trace activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The KI#2 in clause 5.2 of the TR 28.837 describes the problem the gaps in signalling based trace activation and proposed a solution. This CR is normatively proposing the solution described in the TR 28.837 for the KI mentioned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to the procedure of signalling based trace activation has been proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The initial message exchanges before the start of the trace in signalling based activation will not be captured in the trace record. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.622 CR 0313 TS 28.623 CR 0296 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

#### 4.1.2.15	5GC activation mechanism

##### 4.1.2.15.1	UE attached to 5GC via NG-RAN

Figure 4.1.2.15.1.1 illustrates the signaling Trace Session activation procedure in 5GC as part of the Registration procedure:

Figure 4.1.2.15.1.1: Trace activation in 5GC following the Registration procedure

The steps 3-6, 12 and 15 below are parts of the General Registration procedure - see 3GPP TS 23.502 [41] clause 4.2.2.2 for specific details. Present document does not attempt to re-define how General Registration procedure works, but rather illustrates the signaling Trace Activation aspects.

1.	Management system activates Trace Session to the UDM. The following trace control and configuration parameters shall be included in the Trace Activation message:

-	Trace Target: SUPI or IMEISV.

-	Trace Reference.

-	Triggering Events for AMF, SMF, UPF and PCF.

-	Trace Depth.

-	List of NE Types to trace.

-	List of Interfaces for AMF, SMF, UPF, PCF and NG-RAN.

-	Trace Collection Entity IP Address for the file-based trace reporting or Trace Reporting Consumer URI for the streaming trace reporting.

2.	UDM stores the trace control and configuration parameters received from the management system.

3.	UE sends Registration request to NG-RAN node.

4.	NG-RAN node selects appropriate AMF.

5.	NG-RAN node forwards the UE Registration request to the selected AMF.

The NG-RAN node shall buffer the steps 3 – 5 so that if the trace is activated for this call, the buffered information can be added to the trace record. This capability shall be configurable at NG-RAN node.

6.	AMF receives the trace control and configuration parameters information from UDM via Nudm_SDM_Get operation (see step 14 in clause 4.2.2.2.2 and clause 5.2.3.3 of 3GPP TS 23.502 [41]).

7.	AMF stores the trace control and configuration parameters received from the UDM.

8.	AMF starts the Trace Session according to the received configuration.

9.	AMF sends the Start Trace message over NG interface (N2 interface from the 5GC perspective). In case the Start Trace message is not received from the AMF before "N2 PDU Session Request" NAS message to NG-RAN node as described in Step 12 of the figure 4.3.2.2.1-1 in [41], the NG-RAN node shall discard the buffered messages from steps 3 – 5. In case the Start Trace message is received from the AMF before "N2 PDU Session Request" NAS message to NG-RAN node, the NG-RAN node shall copy the buffered content to the trace record and shall discard the buffered content.

10.	NG-RAN node stores the trace control and configuration parameters received from the AMF. This step is part of NG-RAN signaling trace activation - see clause 4.1.2.16 for more details.

11.	NG-RAN node starts the Trace Session according to the received configuration. This step is part of NG-RAN signaling trace activation - see clause 4.1.2.16 for more details.

12.	AMF establishes Policy Association with PCF (see step 16 in clause 4.2.2.2.2 of 3GPP TS 23.502 [41]) and provides the trace control and configuration parameters information to the PCF.

13.	PCF stores the trace control and configuration parameters received from the AMF as part of Policy Association.

14.	PCF starts the Trace Session according to the received configuration.

15.	AMF provides the trace control and configuration parameters information to the SMF as part of the SM Context (see step 18 in clause 4.2.2.2.2 of 3GPP TS 23.502 [41]).

16.	SMF stores the trace control and configuration parameters received from the AMF.

17.	SMF starts the Trace Session according to the received configuration.

End of Changes