9072123GPP TSG-SA5 Meeting #152 	S5-238205

Chicago,US, 13-17 November 2023	(revision of S5-237884)

# Source:	Ericsson

# Title:	New WID on Support for Subscriber and Equipment Trace and QoE collection functionality in RAN and core network

# Document for:	Approval

# Agenda Item:	6.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Support for Subscriber and Equipment Trace and QoE collection functionality in RAN and core network

Acronym:	TRAQ

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  |  |  |
| No | X | X |  |  | X |
| Don't know |  |  | X | X |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The number of automation functions are increasing in the RAN, the core network and in the management system. For them the need for input data on session basis is increasing using QoE, trace and MDT etc.
There are already a number of measurements specified in Rel-18 and earlier, that needs management support:

- Random Access (RA) Report

- Successful Handover Report (SHR)

- Successful PSCell Change Report (SPR)

- Mobility History Information (MHI) Report

- Xn message UE History Information (UHI)

- QoE measurement collection for application sessions delivered via MBS broadcast or multicast.

- QMC support in RRC_IDLE and RRC_INACTIVE.

There are also discussions in RAN regarding new Trace and MDT work for SON for Rel-19 in RP-232624 Moderator’s summary for REL-19 RAN3 topic. If the management support is going to come in the same release as the RAN feature, the coming normative RAN work needs to have a place holder.

SA2 has several approved studies and there is still some more candidate topics for studies and WI. Several of them might impact Trace, MDT and QoE in Rel-19.

# 4	Objective

###### To specify:

- WT#1 Specify use cases and requirements.

- WT#2 Specify overall function descriptions.

- WT#3 Specify updated management requests.

For all WTs:
This is to be done for the already specified Rel-18 RAN MRO measurements:

- Random Access (RA) Report

- Successful Handover Report (SHR)

- Successful PSCell Change Report (SPR)

- Mobility History Information (MHI) Report

- Xn message UE History Information (UHI)

New Rel-19 topics from normative work out of RP-232624 and/or SA2.

Also the RAN QoE work in Rel-18 that does not yet have any management support is included:

- QoE measurement collection for application sessions delivered via MBS broadcast or multicast.

- QMC support in RRC_IDLE and RRC_INACTIVE.

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- | --- |
| WT#1 | 0 | 0.2 | Maybe | Maybe |
| WT#2 | 0 | 0.5 | Maybe | Maybe |
| WT#3 | 0 | 0.3 | Maybe | Maybe |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 32.421 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 32.422 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 32.423 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 28.404 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 28.405 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 28.406 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 28.622 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |
| 28.623 | Align with RAN functionality for Subscriber and Equipment Trace and QMC. | SA#109 (Sep. 2025) |  |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

SA5

# 8	Aspects that involve other WGs

Potential collaborations:

SA2, SA4, CT1, CT4, RAN2 and RAN3.

RAN has proposals for work that might impact Trace, MDT and QoE in RP-232624.

Studies in SA2 might impact Trace, MDT and QoE in Rel-19.

If Signalling Based Activation is affected, CT4, RAN2 and RAN3 are affected.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

Intel

Samsung

Telefonica