3GPP TSG-SA5 Meeting #155 	S5-242929

# Jeju, Korea (Republic Of), 27th May 2024 - 31st May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input To DraftCR TS 28.105 Add lifecycle management operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing mapping of defined IOCs to Operations and/or Notifications that it can support. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to bring clarity on what kind of LCM oparations and noticiations a IOC can support, this contribution proposes to add LCM operation and/or notification mapping table to IOCs defined in this specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | leads to incorrect and incomplete implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of modification

# 8	Service components

The generic provisioning management service in [11] clause 11.11 are used by AI/ML Management.

## 8.1	Lifecycle management operations for AI/ML Management MnS

Thecomponents for ML training MnS are listed in table 8.1-1.

Table 8.1-1: Components for ML training

|  | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| ML training management capability |  |  |  |
| Create an ML training request | createMOI operation | MLTrainingRequest | N/A |
| Modify an ML training request | modifyMOIAttributes operation |  |  |
| Query an ML training request | getMOIAttributes operation |  |  |
| Query an ML training report | getMOIAttributes operation | MLTrainingReport |  |
| Modify an ML training process | modifyMOIAttributes operation | MLTrainingProcess |  |
| Query an ML training process | getMOIAttributes operation |  |  |
| Create, Delete, and Update ML training | changeMOIs operation | MLTrainingRequest MLTrainingReport MLTrainingProcess |  |

The components for ML testing are listed in table 8.1-x.

Table 8.1-x: Components for ML testing

| ML testing management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML testing request | createMOI operation | MLTestingRequest | N/A |
| Modify an ML testing request | modifyMOIAttributes operation |  |  |
| Query an ML testing request | getMOIAttributes operation |  |  |
| Query an ML testing report | getMOIAttributes operation | MLTestingReport |  |
| Query an ML testing report subscription | getMOIAttributes operation |  |  |
| Create, Delete, and Update ML testing | changeMOIs operation | MLTestingRequest MLTestingReport |  |

The components for ML deployment are listed in table 8.1-x.

Table 8.x-1: Components for ML deployment

| ML deployment management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML loading request | createMOI operation | MLEntityLoadingRequest | N/A |
| Modify an ML loading request | modifyMOIAttributes operation |  |  |
| Create an ML loading policy | createMOI operation | MLEntityLoadingPolicy |  |
| Delete an ML loading policy | deleteMOI operation |  |  |
| Modify an ML loading policy | modifyMOIAttributes operation |  |  |
| Query an ML loading policy | getMOIAttributes operation |  |  |
| Modify an ML loading process | modifyMOIAttributes operation | MLEntityLoadingProcess |  |
| Query an ML loading process | getMOIAttributes operation |  |  |
| Create, Delete, and Update ML Loading | changeMOIs operation | MLEntityLoadingRequest MLEntityLoadingPolicy MLEntityLoadingPolicy |  |

The components for ML model inference are listed in table 8.1-x.

Table 8.x-1: Components for AI/ML inference

| AI/ML Inference management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML update request | createMOI operation | MLUpdateRequest | N/A |
| Modify an ML update process | modifyMOIAttributes operation | MLUpdateProcess |  |
| Query an ML update process | getMOIAttributes operation |  |  |
| Query an ML update report | getMOIAttributes operation | MLUpdateReport |  |
| Query an AI/ML inference report | getMOIAttributes operation | AIMLInferenceReport |  |
| Create, Delete, and Update ML Update and ML Inference Report | changeMOIs operation | MLUpdateRequest MLUpdateProcess MLUpdateProcess AIMLInferenceReport |  |

End of modification