3GPP TSG-SA5 Meeting #155 	S5-242978

Jeju, Korea (Republic Of), 27th May 2024 - 31st May 2024

# Source:	Ericsson Hungary Ltd.

# Title:	Rel-19 pCR TR 28.871 Separate subscriptions

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871  Study on Service Based Management Architecture enhancement phase 3

[2]	3GPP TS 28.532	Generic management services

# 3	Rationale

It is a common case, that on the consumer side separate, different SW entities handle different OAM aspects like FM, PM, CM, etc.These different MnS consumer SWs may even belong to different organizations in the operator's management systems. For this reason a producer should be prepared to send the notifications from the different MnS producers (PM, FM, CM) to different destinations.

# 4	Detailed proposal

First change

# 5.a Separate subscriptions

### 5.a.1 Description

There are consumer implementations where different entities handle different OAM aspects like FM, PM, CM, etc. For this reason producer (s) should be prepared to send the notifications from the different MnS producers (PM, FM, CM) to different destinations. Multiple NtfSubscriptionControl MOIs should be supported, at least one per MnS producer (type). This issue becomes even more important if the different MnS consumers are handled by different organizations in the operator's management system.

### 5.a.2 Potential requirements

REQ-Separate-subscriptions-1: It should be possible to send notifications from different MnS producers to different destinations using separate subscription parameters.

### 5.a.3 Potential solutions

#### 5.a.3.1 Potential solution#1 Allow multiple NtfSubscriptionControl MOIs.

An MnS producer should support creating multiple NtfSubscriptionControl MOIs, at least one per MnS producer (type).

If the producer does not support creation and deletion of NtfSubscriptionControl instances by MnS consumers, it should provide multiple NtfSubscriptionControl MOIs created by the system or pre-installed.

The attributes of NtfSubscriptionControl shall be writable even if the MOIs were system-created or pre-installed .

### 5.a.4 Evaluation of potential solutions

TBD

End of change