3GPP TSG-SA5 Meeting #155 	S5-243110

Jeju, Korea, 24 - 28 May 2024

# Source:	China Mobile

# Title:	Rel-19 pCR TR28.9xx Add use case of performance monitoring

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

The group is asked to discuss and approve.

2	References

None.

3	Rationale

The WT-2 in R19 of AIML_MGT is to study the management aspects (LCM CM and PM) of AI/ML functionalities defined by other 3GPP WGs.The performance of the models trained and used in NWDAF is considered to be monitored Therefore, it is necessary to add the use case related with performance monitoring of ML models in NWDAF..

This contribution is to add the use case and potential requirements for management aspects of performance monitoring of ML models in NWDAF.

4	Detailed proposal

It proposes to make the following changes to TR 28.9xy or AI/ML management phase 2.

1st modified section

5	Use cases, potential requirements and possible solutions

5.x	Management Capabilities for ML training phase

5.x.x	Performance Management of ML training

5.x.x.1	Description

TBD.

5.x.x.2	Use cases

5.x.x.2.1	Performance monitoring of ML models in NWDAF

For NWDAF, the ML training function can be located in NWDAF or management system, the AI/ML inference function is in the NWDAF. The performance of ML models trained or used in NWDAF needs to be monitored and evaluated. The related performance indicators of the models in NWDAF need to be collected and analyzed by the management system. In TS28.105, the ModelPerformance data type specifies the performance of an ML model. The attribute performanceScore is associated to the IOC MLEntity, which includes the attribute mLEntityId. However, the model in NWDAF is identified by the model ID or Analytics ID, which is not understood by the management system. Therefore, once the model ID or Analytics ID is associated with the mLEntityId, the management system can use the attribute performanceScore to evaluate the  performance of models in NWDAF.

The management system should have the capabilities of:

-	identifying the model identified by the model ID or Analytics ID in NWDAF;

-	collecting the information related with performance indicators from NWDAF;

-	monitoring and evaluating the performance of ML models in NWDAF.

5.x.x.3	Potential requirements

REQ-PM-NWDAF-1: The ML Training or Testing MnS producer shall have a capability to allow an authorized consumer to query the performance of model trained in NWDAF.

End of modified sections