3GPP TSG-SA5 Meeting #155 	S5-243141

Jeju, South Korea, 27 - 31 May 2024

# Source:	ZTE Corporation

# Title:	Add use case and requirements on CCL for multiple-domain fault management

# Document for:	Approval

Agenda Item:	6.19.4

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TS 28.535: "Management services for communication service assurance; Requirements"

[2]	3GPP TS 28.536: "Management services for communication service assurance; Stage 2 and stage 3"

# 3	Rationale

This contribution proposes to add use case and potential requirements on CCL for multiple domains fault management.

# 4	Detailed proposal

1st Change

# 5. 			Use Cases

5.x			CCL for multiple-domain fault management

5.x.1		Description

In certain scenarios, despite the implementation of CCL automation for fault management, network and service failures may persist. Additionally, CCL for  a certain domain fault management might reveal that the root cause extends beyond its own domain.

For instance, a 5GC service failure may be attributed to RAN NE configuration issues. So 5GC domain MnF  need escalation to  high hierarchy domain MnF CCL. In such cases, the alarms and fault-related analysis specific to 5GC domain MnF  can serve as input for high hierarchy domain MnF CCL consideration.

There are two scenarios:

- High hierarchy domain CCL analyzes alarms and 5GC MnF analysis, then makes decision and executes through provisioning service to NG-RAN MnF to solve the network and service failure.

- If high hierarchy domain CCL cannot fixed the failure, it can also delegate NG-RAN domain MnF to create CCL to solve the failure.

5.x.2		Potential Requirements

REQ-CCLM_ FAULT -01 The 3GPP management system should have the capability to collect domain fault management analysis as input to high hierarchy domain fault management analysis.

REQ-CCLM_ FAULT -02 The 3GPP management system should have the capability to allow management certai domain MnF(5GC domain MnF or NG-RAN domain MnF) CCL escalate network and service failure to high hierarchy domain CCL.

REQ-CCLM_ FAULT -03 The 3GPP management system should have the capability to allow high hierarchy domain CCL delegate network and service failure to a certain domain MnF (5GC domain MnF or NG-RAN domain MnF) CCL.

5.x.3		Potential Solutions

5.x.3.1 	Solution-x

5.x.3.2	    Solution-y

5.x.4		Evaluation of solutions

End of change