3GPP TSG-SA5 Meeting #159	S5-250067

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 Conditional control of CCLs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case, requirements and solution description for Conditional control of CCLs of CCLs |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | use case for Conditional control of CCLs of CCLs will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 (new clause 5.1 added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	Closed Control Loops Coordination Capabilities

## 6.D	Coordinating CCLs with other management functions

### 6.D.1	Description

A CCL makes and executes decisions in different network contexts and for different network functions and parameters. Yet within the network, there may be other management functions or features including MDA functions, SON functions, and AIML Functions, which also make decisions that affect the same network functions and parameters as the CCL. The operation of CCLs needs to be coordinated with the other management functions.

NOTE: This use-case only focuses on coordinating CCLs with other management functions for executing decisions.

For a given context, the CCL should indicate to a coordination functionality the set of network functions and corresponding parameters which it is interested in changing. Accordingly, the coordination functionality may subscribe to changes to the network functions and corresponding parameters. Then in case of changes to subscribed network functions and parameters, the network functions will notify the coordination functionality of these changes. The coordination functionality subsequently notifies the CCL of the information on the latest changes to a network function parameter and an identifier of a management entity/function (e.g. CCL, MDA, SON, AI/ML inference Function) responsible for the change to the parameter.

The CCL determines the history of previous values of the parameter, e.g., from the coordination functionality. The history includes, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. The CCL defines a favourable range of values of the parameter based on the received information on the latest change and the history of previous changes to the parameter. The CCL calculates a new value of the parameter considering the favourable range as a constraint for the new value. The CCL can then update the value of the parameter of the network function to the new value.

### 6.D.2	Requirements

REQ- CCL-ESC-1: The CCL MnS producer should have a capability to indicate to a coordination functionality the set of network functions including their parameters which it is interested in changing.

REQ- CCL-ESC-2: The CCL coordination MnS producer should have a capability to enabling an authorized MnS consumer to receive information on the latest changes to a network function parameter and an identifier of a management entity/function responsible for the change to the parameter.

NOTE 1: an example MnS consumer may be a CCL instance.

NOTE 2: an example of management entity/function responsible for the change to the parameter may include an instance of a MDA Function, a SON Function or an AI/ML inference Function. These functions may not necessarily execute or be responsible for executing the change.

REQ-CCL-ESC-3: The CCL coordination MnS producer should have a capability to enabling an authorized MnS consumer to receive the history of previous values of the parameter, e.g., from the coordination functionality.

NOTE 3: The history includes, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter.

### 6.D.3	Solution Description

First, a CCL needs to indicate to different entities (a coordination functionality) the set of network functions and corresponding parameters which it is interested in changing. For this, the CCL’s control scope is included in the sourceIndicator attribute of the notifyMOIcreation which is sent to functions with which the CCL needs to be coordinated including the coordination entity. Then, when CCL is instantiated, information on its control scopes is notified to the CCL coordination entity and other functions.

The CCL coordination entity monitors action son different network scopes to identify cases where the CCLs control spaces are affected by other management and network functions. Accordingly, the CCL coordination entity contains an attribute for a data type representing information about the control scopes of different CCLs. The CCL’s control scopes data type includes, for each CCL,  an attribute representing the history of changes to that control scope by any other management entity which is not the CCL..

NOTE:	The logging of history may be based on a generic logging data functionality that is not specific to CCLs.

The CCL’s control scopes contains an per-CCL-control-scope attribute, called control scope change representing a change to that control scope by another management entity which is not the intended CCL. The attribute is notifiable, so that in case of a change, the CCL coordination entity writes the change into the control scope change attribute which is then notified to the respective CCL.

Given the notification, the CCL can read the control scope change to obtain the history of changes and trigger subsequent actions if needed.

End Change