3GPP TSG SA5 Meeting #159	S5-250282

Sophia-Antipolis, France, 17 - 21 February 2025

3GPP TSG SA WG5 Meeting #158	S5-246405

Orlando, USA, 18 - 22 November 2024

Source:	RAN2

Title:	Reply to SA5 LS on Number of UEs in RRC_INACTIVE state with data transmission

Document For:	Information

Agenda Item:	6.1

3GPP TSG-RAN WG2 Meeting #127bis	   R2-2409280

Hefei, China, 14 - 18 October, 2024

Title:	Reply to SA5 LS on Number of UEs in RRC_INACTIVE state with data transmission

Response to:	S5-245138

Release:	Rel-19

Work Item:	PM_KPI_5G_Ph4

Source:	RAN2

To:	SA5

Cc: 	RAN3

Contact Person:

Name:	Dawid Koziol

Email:		dawid.koziol AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission. RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session as the connection requests from such UEs are distinguished either based on dedicated RACH resources or based on a dedicated RRC resume cause. However, RAN2 would also like to inform SA5 that occasionally an SDT procedure may be unsuccessfully completed due to cell re-selection during SDT procedure, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold or integrity check failure while SDT procedure is ongoing. In these cases, the network would not be aware that the UEs SDT session is already terminated at the UE side, but would consider SDT session as ongoing until timer T319a expires. T319a is started when the SDT session starts and its maximum value is 4 seconds.

2. Actions:

To SA5:

ACTION: RAN2 respectfully asks SA5 to take the above into consideration in their further work and inform RAN2 if and which L2 metrics SA5 would like RAN2 to specify.

3. Date of Next RAN2 Meetings:

RAN2#128	18th – 22nd November 2024			Orlando, US

RAN2#129	17th – 21st February 2025				Athens, Greece