3GPP TSG-SA5 Meeting #159	S5-250554

Sophia-Antipolis, FR, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0068 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Add information model definitions for energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 22.261 requires that energy related information, including ratio of renewable energy and carbon emission information for the 5G system, be measured. The Energy Information Function (EIF) newly introduced into TS 23.501 may consume not only gNB and UPF Energy Consumption and Data Volume measurements from OAM, but also other energy related information such as carbon emission and renewable energy factor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new energy related information into TS 28.310. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EIF cannot get the expected energy related information about gNBs and UPFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[3]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface".

[4]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[5]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[6]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[7]	Void.

[8]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[9]	3GPP TS 32.435: "Telecommunication management; Performance measurement; eXtensible Markup Language (XML) file format definition".

[10]	3GPP TS 32.436: "Telecommunication management; Performance measurement: Abstract Syntax Notation 1 (ASN.1) file format definition".

[11]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[12]	3GPP TS 38.401: "NG-RAN; Architecture description".

[13]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[14]	Void.

[15]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[16]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[17]	3GPP TS 32.551: "Energy Saving Management (ESM); Concepts and requirements".

[18]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[19]	ETSI GR NFV-IFA 015 V3.4.1 (2020-06): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Report on NFV Information Model".

[20]	ETSI GR NFV 003 V1.6.1 (2021-03): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[21]	3GPP TS 28.530: " Management and orchestration; Concepts, use cases and requirements".

[22]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[23]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[24]	Void.

[25]	ETSI GS NFV-IFA 027 (V4.3.1) (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[26]	ETSI ES 202 336-12 (V1.2.1) (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[27]	ETSI GS NFV-IFA 008 V3.5.1 (2021-11): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[28]	ETSI GS NFV-TST 008 V3.3.1 (2020-12): "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification".

[29]	ETSI GS NFV-IFA 006 (V3.7.1) (2022-11): " Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Vi-Vnfm reference point – Interface and Information Model Specification".

[30]	3GPP TS 28.111: "Fault management".

[x]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[z]	ITU-T L.1333: "Carbon data intensity for network energy performance monitoring".

[t]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

Next change

# X	Information model definitions for NR NRM

## X.1	Imported and associated information

### X.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC, SubNetwork | SubNetwork |
| TS 28.622 [y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [y], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [y], IOC, Top | Top |

### X.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |

## X.2	Class diagram

### X.2.1	Relationships

Figure X.2.1-1: Co-located Network Functions group NRM fragment 1

Figure X.2.1-2: Co-located Network Functions group NRM fragment 2

Figure X.2.1-3: Energy related information NRM fragment

### X.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure X.2.2-1: Co-located Network Functions group NRM fragment

Figure X.2.2-2: Energy supply and energy source information NRM fragment

Figure X.2.2-3: Energy supply type and energy source type NRM fragment

## X.3	Class definitions

### X.3.1	ColocatedNFsGroup

#### X.3.1.1	Definition

This IOC represents a grouping of network elements or network functions co-located on the same operator site. Co-location refers to the sharing of the same type(s) of energy supply by the group of network elements or network functions.

This attribute memberMEList contains the DNs of managed entities that are members of the group

#### X.3.1.2	Attributes

The ColocatedNFsGroup IOC includes the attributes inherited from Top IOC and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| memberMEList | M | T | T | F | T |

#### X.3.1.3	Attribute constraints

None

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	EnergySupplyInfo

#### X.3.2.1	Definition

This IOC defines the information related to energy supply.

The attribute energySupplyTypeRatio indicates the percentage of energy supplied from a type of energy supply. The attribute energySupplyTypeRef is used to refer to the type of energy supply.

#### X.3.2.2	Attributes

The EnergySupplyInfo IOC includes the attributes inherited from Top IOC and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySupplyTypeRatio | M | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| energySupplyTypeRef | M | T | T | F | T |

#### X.3.2.3	Attribute constraints

None.

#### X.3.2.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.3	EnergySupplyType

#### X.3.3.1	Definition

This IOC defines the type of energy supply.

The attribute energySupplyTypeUserLabel indicates the means by which energy is supplied to the managed entity.

The attribute energySupplyInfoRef is used to refer to a type of energy supply.

#### X.3.3.2	Attributes

The EnergySupplyType IOC includes the attributes inherited from Top IOC and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySupplyTypeUserLabel | M | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| energySupplyInfoRef | O | T | T | F | T |

#### X.3.3.3	Attribute constraints

None.

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.4	EnergySourceInfo

#### X.3.4.1	Definition

This IOC defines the information related to a type of energy source.

The attribute energySourceRatio represents the proportion of the energy provided by an energy source type relatively to the total energy supplied to the ManagedEntity through an energy supply type.

The attribute energySourceTypeRef contains the DN of EnergySourceType.

#### X.3.4.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySourceRatio | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| energySourceTypeRef | M | T | T | F | T |

#### X.3.4.3	Attribute constraints

None.

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.5	EnergySourceType

#### X.3.5.1	Definition

This IOC defines the information related to a type of energy source.

The attribute energySourceTypeUserLabel indicates the type of energy source used to supply energy to the managed entity.

The attribute energySourceCef represents the Carbon Emission Factor of the type of energy source.

The attribute renewableEnergyFactor represents the percentage of renewability of the energy source type.

#### X.3.5.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySourceTypeUserLabel | M | T | T | F | T |
| energySourceCef | M | T | T | F | T |
| renewableEnergyFactor | O | T | T | F | T |

#### X.3.5.3	Attribute constraints

None.

#### X.3.5.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

## X.4	Attribute definitions

### X.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| memberMEList | This attribute contains the DNs of managed entities that are members of the group. allowedValues: DN of any MOI. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySupplyTypeRatio | The proportion of the energy provided by the energy supply type relatively to the total energy supplied to the ManagedEntity. Allowed values: integer between 0 and 100. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| energySupplyTypeRef | This attribute contains the DNs of EnergySupplyInfo. allowedValues: Not applicable. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySupplyTypeUserLabel | The means by which energy is supplied to a managed entity. It can be one of the following: - GRID_ELECTRICITY: the energy consumed comes from local grid electricity (See Grid electricity in clause 6 of ITU-T Recommendation L.1333 [z]) - BACKUP_ENERGY: the energy comes from backup diesel or other types, or from generators such as solar panels. (See Backup energy in clause 6 of ITU-T Recommendation L.1333 [z]) - LOCALLY_GENERATED_ENERGY: energy provided by local generators not used for backup functionality, e.g., solar, wind generator. (See Locally generated energy in clause 6 of ITU-T Recommendation L.1333 [z]) AllowedValues: - “GRID_ELECTRICITY” - “BACKUP_ENERGY” - “LOCALLY_GENERATED_ENERGY” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyInfoRef | This attribute contains the DN of EnergySupplyType. allowedValues: Not applicable. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySourceRatio | The proportion of the energy provided by the energy source type relatively to the total energy supplied to the ManagedEntity through an energy supply type. Allowed values: integer between 0 and 100 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| energySourceTypeRef | This attribute contains the DN of EnergySourceType. allowedValues: Not applicable. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySourceTypeUserLabel | The type of energy source. Allowed values: N/A When the energy source type values are based on ETSI GS OEU 020 [a], this attribute shall follow the following pattern: "etsi:<energy source>". For example, the values can be "etsi.Motor Gasoline", "etsi: Anthracite" etc. When the energy source type values are not based on ETSI GS OEU 020 [t], this attribute shall follow the following pattern: "other:<energy source>". For example, the values can be "other.solar", "other:wind", etc. Example values (from ETSI GS OEU 020 [t]): "etsi:Motor Gasoline", "etsi:Anthracite, "etsi:Residual Fuel Oil, "etsi:Other Bituminous Coal", "etsi:Sub-Bituminous Coal", "etsi:Lignite", "etsi:Natural Gas", "etsi:Municipal Wastes (non-biomass) ", "etsi:Sustainable wood", "etsi:Unsustainable wood, Plant oil", "etsi:Biodiesel", "etsi:Bioethanol", "etsi:Solar Thermal", "etsi:Geothermal", "etsi:Solar PV", "etsi:WindPower", "etsi:HydroPower". Other sources than ETSI GS OEU 020 [t] may be used by the Operator. Which source is to be used by the Operator and how the Operator gathers this information are out of scope of the present document. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySourceCef | The Carbon Emission Factor of a type of energy source. Its unit is kg CO2eq / kWh. Allowed values: non-negative integers Example values (see ETSI GS OEU 020 [t]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| renewableEnergyFactor | This indicates the percentage of renewability of the energy source type. allowedValues: integer between 0 and 100. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |

## X.5	Common notifications

### X.5.1	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [16], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

Table 9.6.1-1

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

End of change