3GPP TSG-SA5 Meeting #159	S5-250710

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	Solution update for topic 3

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

# 3	Rationale

Update of solution architecture update as well as correcting missing references and references to wrong potential requirements.

# 4	Detailed proposal

First change

#### 6.4.1.1	Use Case #4.1: MNO charges SCC

This use case focuses on SCC and MNO business scenario.

An SCC has a subscription with an MNO which rents the satellite from an SSP, to allow MNO to provide UE-sat-UE to its subscribers.

The charging party and charged party can be:

-	Charged party: the SCC identified by the UE.

-	Charging party: MNO.

The MNO charges the subscribers based on the usage of satellites to provide the UE-sat-UE communication service.

Potential charging requirements: REQ- CH_ SAT_PH3-01.

Second change

#### 6.4.1.2	Use Case #4.2: MNO charged by SSP

This use case focuses on MNO and SSP business scenario.

An MNO has a wholesale agreement to use the satellite from an SSP.

The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP

The SSP charges MNO based on the usage of satellites.

Potential charging requirements: REQ- CH_ SAT_PH3-01.

Third change

##### 6.3.4.1.2	Architecture description

Figure 6.3.4.1.2-1: Satellite resource usage charging architecture

The SMF and CEF interact with CHF using Nchf interface. The CHF produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of charging data per UE for an SSP. The wholesale part is only included for completeness and is currently outside the scope of 3GPP SA5.

SMF collect charging data related to satellite backhaul (satellite backhaul category, data volumes) for each subscriber according to TS 32.255 [8].

CEF collect charging data related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes) according to TS 32.257 [x].

End of changes