3GPP TSG-SA5 Meeting #159	S5-250803

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR Rel-19 TS 28.105 Use case on federated learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Intel, NEC, ZTE, Ericsson, Samsung, China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support enabling the ML training MnS consumer to manage federated learning aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a new use case and sever requirements on management aspects of Federated learning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for ML training MnS consumer to manage federated learning aspects, e.g., HFL among NWDAFs in SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.2, 6.2b.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: a manageable representation of an ML model algorithm.

NOTE 1:	an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

NOTE 2:	ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

NOTE 3:	ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

ML model initial training: a process of training an initial version of an ML model.

ML model re-training: a process of training a previously trained version of an ML model and generate a new version.

NOTE 4:	a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

ML model joint training: a process of training a group of ML models.

ML training function: a logical function with ML model training capabilities.

ML model testing: a process of evaluating the performance of an ML model using testing data different from data used for model training and validation.

ML model joint testing: a process of evaluating the performance of a group of ML models using testing data different from data used for model training and validation.

ML testing function: a logical function with ML model testing capabilities.

AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

NOTE 5:	the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

ML model deployment: a process of making a trained ML model available for use in the target environment.

ML model loading: a process of making a trained ML model available to an inference function.

AI/ML activation: a process of enabling the inference capability of an AI/ML inference function.

AI/ML deactivation: a process of disabling the inference capability of an AI/ML inference function.

Federated Learning:  a distributed machine learning approach where the ML model is trained collaboratively by multiple ML training functions. This includes multiple FL clients, which perform training on local data, and one FL server, which aggregates model outcomes from the clients iteratively without exchanging data samples.

Horizontal Federated Learning:  a federated learning technique without exchanging/sharing local data set, wherein the local data set in different HFL clients for local model training have the same feature space for different samples.

FL Client: a training function that trains an ML model on local data and shares only the model updates with the FL server/FL Client, preserving data privacy.

FL Server: a function that aggregates the ML model updates from FL Clients to produce a global ML model.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and TS 28.533 [15]. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 28.533 [15].

AI	Artificial Intelligence

ML	Machine Learning

FL	Federated Learning

HFL	Horizontal Federated Learning

VFL	Vertical Federated Learning

Start of Next change

# 6	AI/ML management use cases and requirements

## 6.2b	ML model training

### 6.2b.1	Description

Before an ML model is deployed to conduct inference, the ML model algoritm associated with the ML model needs to be trained. The ML model training can be an initial training or the re-training of an already trained ML model.

The ML model is trained by the ML training MnS producer, and the training can be triggered by request(s) from one or more ML training MnS consumer(s), or initiated by the ML training MnS producer (e.g., as a result of model performance evaluation).

### 6.2b.2	Use cases

#### 6.2b.2.X	Management of Federated Learning

#### 6.2b.2.X.1	Description

Federated Learning (FL) is a distributed machine learning approach that allows multiple FL clients to collaboratively train an ML model on local datasets contained in each FL Client without explicitly exchanging data samples.

FL is supported by a group of FL clients and FL server wherein FL client keeps the data localized and private and trains the ML model directly on the local nodes (client) where the data is obtained or stored.

Federated learning can be categorized into two main types: Horizontal federated learning (HFL) and Vertical Federated Learning (VFL), based on the nature of the data distribution and the way the model training is orchestrated among participants. For HFL, the process typically includes FL Client discovery and selection, local ML model training and updates by the FL Clients, ML model updates aggregation, and global ML model distribution by the FL Server. Management of Federated learning can be used for AI/ML-based use cases specified in [2] and [3].

Figure 6.2b.2.X.1-1: ML model distribution and aggregation for HFL

NOTE:	A prior agreement as well as authentication procedures needs to be established between FL Server and FL clients before sharing any information between these functions.

### 6.2b.2.X.2			Use cases

##### 6.2b.2.X.2.Y			Management of different roles in Federated Learning

For FL, an ML model is collaboratively trained by a group of FL clients (e.g. MTLF in NWDAFs) including one acting as FL server and the others acting as FL clients. Federated Learning training allows multiple FL clients to collaboratively train an ML model on local datasets..

For managing the FL, the ML training MnS consumer needs to know the FL clients and FL server involved in the FL, so that the consumer understands the impact of each one of them and can manage them correspondingly.

When receiving an ML Training request, the ML Training MnS Producer should evaluate whether FL process needs to be started according to the training requirements (e.g. minimum number of FL Clients, minimum number of total iterations, minimum number of data samples and available time of the FL Clients, fault tolerance, energy source and carbon emission information) provided by the ML training consumer. Based on the received requirements, the ML Training MnS Producer with the role of FL server may select (including adding and removing) appropriate FL Clients.

To evaluate the performance of FL, the consumer can query the performance of the final global ML model running on the local training data set of participating FL clients. For instance, if an FL server cannot produce a global ML model with satisfied performance for the FL clients, the consumer may interact with the MnS ML training producer to optimize the FL for future training, e.g. updating the criteria for selecting FL clients.

In addition, the consumer needs to get the information about the contribution of each FL client to the FL process, for instance, the number of iterations in which the FL client participated in the FL, the number of data samples the FL client used and thetraining duration performed by the FL Client.

Start of Next change

### 6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_FL-1 | The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning. | Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y) |
| REQ-ML_TRAIN_FL-2 | The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer. | Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y) |
| REQ-ML_TRAIN_FL-3 | The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting (including adding and removing) FL clients in Federated Learning to the MnS Producer. | Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y) |
| REQ-ML_TRAIN_FL-4 | The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of the global ML model on each participating FL Client. | Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y) |
| REQ-ML_TRAIN_FL-5 | The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer. | Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y) |
| REQ-ML_TRAIN_FL-6 | The ML training MnS producer should have a capability to report the candidate FL Clients for the FL process. | Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |  |  |

End of changes