3GPP TSG-SA5 Meeting #160	S5-251255

Goteborg, Sweden, 07 - 11 April 2025

Source:	Nokia, Ericsson

Title:	Pseudo-CR on NDTs for verification

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.1.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to Add a description of of NDT use cases and requirements for verification

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.3	NDT support for verification - NDTVER

### 5.3.X	Description

This describes NDT use in support of verification.

### 5.3.Y	Use cases

#### 5.3.Y.1	General verification - GEN

To evaluate a network scenario, the NDT needs to be configured with data defining a network scenario, and where applicable, the configurations and functionalities to be evaluated. The MnS consumer should be enabled to configure the NDT accordingly. Given the scenario, configurations and functionalities, the consumer can then control the simulation (e.g. to start or stop the simulation) according to the LCM capabilities as described in clause 5.3. The NDT simulates the defined scenario configurations and functionalities; and collects performance metrics indicating the impact of the configurations and functionalities, which are then compiled into an analysis report. The MnS consumer should be enabled to configure the information that should be included in the simulation report. The NDT MnS producer should be enabled to send the simulation report to MnS consumer to indicates the network simulation results.

#### 5.3.Y.1	Verification of network response to events - EVT

The network can experience different kinds of events, including failure and non-failure events. A failure event refers to the situation where users in the network are completely unable to get service from the network. A non-failure event refers to the situation where the network experiences an unusual occurrence but users are still able to get service. The NDT can be used to evaluate how the network responds any of these events. The NDT can model network behaviors and provide information on potential impact of network events. The NDT can be used to evaluate if a particular event will cause a network failure or whether the network can withstand the event.

The following are examples of events that can be supported:

- 	Signaling storms: A signaling storm is a failure events where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm. For a signaling storm, the NDT provides information on potential impact of surges in signaling requests on the network. The information can enable the MnS consumer to identify the optimal flow control parameters for each signaling impact point.

#### 5.3.Y.2	Verification of network configurations- NETCFG

Several configurations can be provided to the network, either as modifications to single parameters or as bulk configurations, e.g., as a configuration plan. There may be multiple configurations that are iteratively executed in NDT until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets the operators’ requirements. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated as well as the input information that should be applied to automation functionality to be evaluated. The information may include configurations, simulated network scenarios or simulated traffic patterns.

The configurations can be evaluated for any event including those listed in 5.3.Y.1

#### 5.3.Y.3	Verification of automation configurations - AUTCFG

Besides network configurations, network automation functionality may also be configured, e.g. configurations of SON functionality say for energy savings. The NDT can be used to evaluate the configurations of the automation functionality. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated for one or more automation functionality with the corresponding input information that should be applied to automation functionality to be evaluated. The information may include configurations, simulated network scenarios or simulated traffic patterns.

The configurations of automation functionality can be evaluated for any event including those listed in 5.3.Y.1

### 5.Z	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTVER-GEN-01: | The management system should support a capability enabling the MnS consumer to define the data that should be included in the report/output on the simulation/emulation of verification of network scenarios and configurations. | NDTVER-GEN |
| REQ-NDTVER-EVT-01: | The management system should have a capability enabling the MnS consumer to define one or more network events to be modelled and evaluated by the NDT. | NDTVER-EVT |
| REQ-NDTVER-EVT-02: | The management system should have the capability to report the simulation/emulation outcomes on the validation of one or more network events to be modelled and evaluated by the NDT | NDTVER-EVT |
| REQ-NDTVER- NETCFG -01: | The management system should have a capability enabling the MnS consumer to define a set of network configurations to be modelled and evaluated against a set of one or network events | NDTVER-NETCFG |
| REQ-NDTVER- NETCFG -02: | The management system should have the capability to report the simulation/emulation outcomes on the validation of a set of network configurations modelled and evaluated by the NDT against one or more network events to be | NDTVER- NETCFG |
| REQ-NDTVER- AUTCFG-01: | The management system should have a capability enabling the MnS consumer to define a set of automation functionality and configurations for the automation functionality to be modelled and evaluated against a set of one or network events. | NDTVER- AUTCFG |
| REQ-NDTVER- AUTCFG-02: | The management system should have the capability to report the outcomes on the validation of a set of automation functionality and configurations modelled and evaluated by the NDT against one or more network events to be | NDTVER- AUTCFG |

* * * End of Changes * * * *