3GPP TSG-SA5 Meeting #160	S5-251357

Goteborg, Sweden, 07 - 11 April 2025

# Source:	ZTE Corporation

# Title:	Input to DraftCR Rel-19 TS 28.105 Update the UC and Req of Reinforcement Learning

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

The Use case for RL is proposed and agreed in SA5 #159. This contribution proposes to update the use case and requirements to differentiate the management support for different RL management scenarios.

There are two typical management scenarios for RL. Scenario 1 -  both the RL Training Component and RL Inference Component are co-located (e.g., both RL agent training and RL agent inference components are located in OAM), and Scenario 2 - the RL agent Training Component and RL agent inference Component are distributed (e.g., RL agent training component is located in OAM, but RL agent inference component is in the gNB).

- For Scenario 1 (both RL training and Inference components located in OAM), the RL actions are generated in OAM and will be further delivered to configure managed entities (e.g., gNB). In this case, the RL environment only include the entities impacted by the actions of the RL agent (RL agent inference component in the OAM). The consumer needs to restrict the scope to implement the RL actions.

- For Scenario 2 (RL agent training component located in OAM, but RL agent inference component in the gNB), since the RL Inference component is in the gNB, the consumer should be allowed to specify the scope where gNBs can play the role of RL agent Inference Component.

In the agreed UC in the Draft CR, it states that a consumer may want to provide a scope to aid the producer to select/create the environment that may include information of the entities the RL agent would affect using the RL model. It implies that the scope includes both the entities directly involved in RL process and other entities impacted by RL agent actions. However, it is not explicit and may be questioned by some companies.

To identify the difference of the SCOPE in these two scenarios above, this contribution proposes to enhance the UC and REQ to reflect the difference explicitly.

4	Detailed proposal

Start of the First Change

6.2b.2.x	Management of Reinforcement Learning

6.2b.2.x.1	Description

In RL, the agent uses a trial-and-error approach to develop a policy that maximises its cumulative reward over time.

RL can be used for supporting various AI/ML functions (including NG-RAN AI/ML supported functions, e.g., Energy Saving (ES), Mobility Load Balancing (MLB) and Mobility Robustness Optimization (MRO), etc.) when the ML training function is located in OAM. For these scenarios, RL needs to be managed.

An RL agent functionally consists of two main parts: a training component and an inference component.

The training component learns to generate or update the RL model from the outcomes of its actions taken by the inference component in an RL environment (e.g., a live 5G subnetwork or a simulated environment). RL can occur in one of two modes for:

-	RL in online mode: in online mode, the RL model is trained and applied in real time through direct interaction between the RL agent components and the RL environment. In this approach, the RL agent’s training component continuously learns by receiving rewards, and observing state transitions resulting from its actions, while the RL agent’s inference component uses the trained model in real time to determine actions in the RL environment.

An RL model trained using online RL in a simulated environment may be deployed in a live network for inference, with or without continued online learning. If online RL is discontinued in the live network, the RL agent inference component in the live network serves the same role as the RL agent inference component in offline RL, as depicted below.

-	RL in offline mode: In offline mode, instead of direct interaction, the training process relies on a pre-collected dataset from a data collection entity. This dataset consists of state transition, action, and reward information collected over a period of time from the RL environment, allowing the model to learn without real-time interaction with the environment.

6.2b.2.x.2	Use cases

6.2b.2.x.2.1	Enabling Reinforcement Learning

Once the RL model is trained and deployed (if training and inference occur on separate entities), the inference component adopts the RL model and makes decisions based on state transitions in the RL environment.

To enable, facilitate, and manage RL for 5GS, the 3GPP management system needs to support the following RL components in performing their respective functions:

-	RL agent training component: collects the data for training, train the RL model and report the training result of the training component. The data for training includes state transitions, rewards from the RL environment for online RL, or the pre-processed data set for a period of time from the data collection entity for offline RL, as well as the actions taken by the RL agent inference component. The trained RL model would be used by the inference component to determine actions for observed state conditions. Additionally, the RL agent training component needs to indicate the RL environment(s) (e.g., a live subnetwork or a simulation subnetwork) for which an RL model has been trained.

-	RL agent inference component: activates the trained RL model (RL policy), receives state transitions from the RL environment, and determine actions based on the RL model. The RL agent inference component needs to be configured with the information about the RL environment(s) where a trained RL model can be used.

-	RL environment: executes the actions determined by the RL agent inference component and reports rewards to the RL agent training component for online RL or to the data collection entity for offline RL.

6.2b.2.x.2.2	Exploration in Reinforcement Learning

Reinforcement Learning (RL) has the ability to learn and adapt itself to dynamic environments and thus finds the near optimal solution for a problem. However, the potential negative impact to the mobile network caused by RL is still the main drawback. In particular, during the exploration step performing trials and learning from errors may have an impact on the operational network and may result in unsafe operations causing network performance degradations. Therefore, the exploration step in RL needs to be under a controlled configuration range that the RL agent is allowed to act upon, so that the RL actions do not violate system performance requirements. If the RL agent behaves in an unexpected manner, there needs to be a set of fall-back actions in place, e.g. to switch from RL-based solution to non-RL-based solution, to fall back to last discrete time step, and to terminate the RL process.

When RL is supported, a consumer may want to provide a scope (e.g. geographical area, time window) that can aid the producer to select/create the environment when performing RL. The environment may include information of the entities the RL agent would affect using the RL model, i.e., the scope dose not only include the entities directly involved in RL process, but also includes other entities impacted by RL agent actions. If the MnS producer supports multiple types of environment, the consumer may want to state their preference for environment type for RL during training, i.e., simulated environment or live network. When the live network is preferred by the MnS consumer, the consumer can provide network performance requirements (e.g. lower bound threshold, acceptable range, maximum performance deterioration Rate, etc.) of performing ML training of RL, so as to make the MnS producer adapt the training configurations to meet the network performance requirements. Furthermore, the consumer can provide its concerned performance metrics to guide the producer to set reward/state.

NOTE:	 Support for both environment types can be considered optional in the RL training.

End of the First Change

Start of the Second Change

6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-MLKTL-1 | The 3GPP management system should have a capability to enable an authorized MnS consumer to discover or request all or part of the available shareable knowledge at a given MLKTL MnS producer. | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.1) |
| REQ-MLKTL-2 | The 3GPP management system should have a capability for an MLKTL MnS producer to provide to an authorized MnS consumer all or part of its available shareable knowledge | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.1) |
| REQ-MLKTL-3 | The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKTL MnS producer to initiate and execute a transfer learning instance to a specified ML model or ML-enabled function | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-MLKTL-4 | The 3GPP management system should have a capability to enable an authorized MnS consumer to manage or control the knowledge request or the knowledge process or transfer learning process, e.g. to suspend, re-activate or cancel the ML Knowledge Request; or to adjust the description of the desired knowledge NOTE: An example MnS consumers include an operator or the function that generated the request for available Knowledge | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-MLKTL-5 | The 3GPP management system should have a capability to enable an ML model or ML training function to register available knowledge to a shared knowledge repository, e.g. through a ML Knowledge Registration process | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-MLKTL-6 | The 3GPP management system should have a capability enabling an authorized MnS consumer to request the Knowledge Repository to provide some or all the knowledge available for sharing based on specific criteria | ML-Knowledge-based Transfer Learning (clause 6.2b.2.X.2.2) |
| REQ-ML_TRAIN_CLUSTER-01 | The ML Training MnS producer should have a capability for an authorized MnS consumer to request training of a cluster of ML models as per clustering criteria associated to a set of multiple contexts from a previously trained ML model. | ML model training for multiple contexts (clause 6.2b.2.X) |
| REQ-ML_TRAIN-PRE-01 | The ML training MnS producer should have a capability allowing an authorized ML training MnS consumer to request pre-specialized training of a ML model. | ML Pre-specialized training (clause 6.2b.2.X) |
| REQ-ML_TRAIN_DST-01 | The 3GPP management system should enable an authorized consumer to provide information on the training dataset distribution. | Training data statistics (clause 6.2b.2.X) |
| REQ-ML_TRAIN_DST-02 | The 3GPP management system should enable an authorized consumer to provide information on the usage of outliers in the training dataset. | Training data statistics (clause 6.2b.2.X) |
| REQ-ML_DIST-TRNG-01 | The ML training MnS producer should have a capability allowing and authorized consumer to provide distributed training requirements to the MnS Producer. | Management of Distributed training (clause 6.2b.2.X) |
| REQ-ML_TRAIN_FL-1; The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-2; The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-3; The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting (including adding and removing) FL clients in Federated Learning to the MnS Producer.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-4; The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of the global ML model on each participating FL Client.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-5; The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-ML_TRAIN_FL-6; The ML training MnS producer should have a capability to report the candidate FL Clients for the FL process.; Management of different roles in Federated Learning (Clause 6.2b.2.X.2.Y). REQ-RL_TRAIN_01; The ML training MnS producer should have a capability allowing an authorized MnS consumer to query if RL training is supported.; Enabling Reinforcement Learning (6.2b.2.x.2.1). REQ-RL_TRAIN_02; The ML Training MnS producer for RL agent training component shall have a capability to report RL types (i.e., online RL, offline RL) to an authorized consumer.; Enabling Reinforcement Learning (6.2b.2.x.2.1). REQ-RL_TRAIN_03; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to get the type and scope of the RL environment for which an RL model has been trained.; Exploration in Reinforcement Learning (6.2b.2.x.2.2). REQ-RL_TRAIN_04; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to select the type of the RL environment for which an RL model is to be trained.; Exploration in Reinforcement Learning (6.2b.2.x.2.2). REQ-RL_TRAIN_05; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to provide the scope of the RL environment for which an RL model is to be trained.; Exploration in Reinforcement Learning (6.2b.2.x2.2). REQ-RL_TRAIN_06; The ML training MnS producer should have a capability allowing an authorized MnS consumer to provide network performance requirements of performing RL training.; Exploration in Reinforcement Learning (6.2b.2.x2.2). REQ-RL_TRAIN_07; The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the configuration range that the RL agent is allowed to explore.; Exploration in Reinforcement Learning (6.2b.2.x2.2). REQ-RL_TRAIN_08; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to provide the scope of the RL environment for the entities impacted by the RL actions.; Exploration in Reinforcement Learning (6.2b.2.x2.2). NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |

End of the Secondt Change