3GPP TSG-SA5 Meeting #160	S5-251381

Goteborg, SE, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.130 | CR | 0048 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.130 Separate business-level requirements from specification-level requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NetShare_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some requirements (REQ-NS_NG-CON-6, REQ-NS_NG-CON-7 and REQ-NS_NG-CON-8) are placed in clause 5 (Business-level requirements) though they are specification-level requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move REQ-NS_NG-CON-6, REQ-NS_NG-CON-7 and REQ-NS_NG-CON-8 to clause 6 (Specification-level requirements). In clause 5, convert REQ-NS_NG-CON-6, REQ-NS_NG-CON-7 and REQ-NS_NG-CON-8 to business-level requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements REQ-NS_NG-CON-6, REQ-NS_NG-CON-7 and REQ-NS_NG-CON-8 would stay misplaced and would have no corresponding business-level requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.6	Requirements for the OAM&P of shared NG-RAN

REQ-NS_NG-CON-1		The 3GPP management system of the MOP shall support a capability to manage both the non-shared network elements and shared network elements in a NG-RAN network at the same time.

REQ-NS_NG-CON-2		The 3GPP management system of the MOP shall support a capability to configure the NG-RAN network element to start the sharing of one currently non-shared NG-RAN network element or stop the sharing of one currently shared NG-RAN network element.

REQ-NS_NG-CON-3		The 3GPP management system of the MOP shall support a capability to configure the POP-specific attributes of the shared NG-RAN individually based on the POPs’ requirements.

REQ-NS_NG-CON-4		The 3GPP management system of the MOP shall have the capability to configure the radio resources partitioning policies for the POPs based on the agreement between POPs.

REQ-NS_NG-CON-5 	      The 3GPP management system of the MOP shall have the capability to configure the administrative state of the operator specific NR Cell DU.

REQ-NS_NG-CON-6         The 3GPP management system of the MOP should have the capability to support access request to the MOP's management system.

REQ-NS_NG-CON-7         The 3GPP management system of the MOP should have the capability to provide access control to obtain configuration, performance and alarm data.

REQ-NS_NG-CON-8         The 3GPP management system of the MOP should have the capability to provide customized access privileges for each POP.

Next change

# 6	Specification level requirements

## 6.1	Requirements

REQ-NS_GEN-FUN-1		The IRPAgent shall support a capability allowing the IRPManager to configure which POPs share each cell.

REQ-NS_GEN-FUN-2		The IRPAgent shall support a capability allowing the IRPManager to know which POPs share each cell.

REQ-NS_GEN-FUN-3		In GWCN, the IRPAgent shall support a capability allowing the IRPManager to configure which POPs share each core network element.

REQ-NS_GEN-FUN-4		In GWCN, the IRPAgent shall support a capability allowing the IRPManager to know which POPs share each core network element.

REQ-NS_PM-FUN-1		The IRPAgent shall have the capability to support subscription of UL and DL data volume measurements per QoS profile criteria for cross operator accounting purpose from the IRPManager.
The QoS profile criteria may include one or more of the following criteria: one QCI indicator, one GBR Indicator, one ARP Indicator, where:

-	a QCI Indicator identifies one specific QCI value. If the indicator is not set then all QCI values should be taken into account.

-	 a GBR Indicator identifies one GBR range value defined by the management system. If the indicator is not set then all GBR ranges should be taken into account.

-	 an ARP Indicator identifies one ARP priority value. If the indicator is not set then all ARP priority values should be taken into account.

REQ-NS_PM-FUN-2		The IRPAgent shall support the IRPManager setting the reliability for the counter instances in a measurement job. The detailed definition of reliability is vendor specific.

REQ-NS_PM-FUN-3		The IRPAgent shall support up to a maximum number of 200 recorded counter instances per granularity period of data volume measurements for cross operator accounting purpose.

REQ-NS_PM-FUN-4		The IRPAgent shall support the IRPManager to access a file containing data volume measurements for cross operator accounting purposes every granularity period.

REQ-NS_GEN-FUN-5         The MnS producer of MOP should support MnS consumer to send access request to the MOP's management system.

REQ-NS_GEN-FUN-6         The MnS producer of MOP should provide access control to obtain configuration, performance and alarm data.

REQ-NS_GEN-FUN-7         The MnS producer of MOP should provide customized access privileges for each POP as MnS consumers.

End of change