3GPP TSG-SA5 Meeting #160	S5-251448

Goteborg, Sweden, 07 - 11 April 2025

# Source:	ZTE Corporation

# Title:	Rel-19 pCR TS 28.561 Add Requirements for Network failure and risk prediction

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

Add Requirements for Network failure and risk prediction use case.

4	Detailed proposal

Start of First Changes

5.2.2.2	Network failure and risk prediction – NFARP

Each operations for network optimization and maintenance on mobile network may cause potential network failures and risks, especially high-risk operations, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching, which may cause network congestion and network breakdown. Operators cannot carry out the high-risk network operations in the physical network directly network optimization. Therefore, utilization of NDT is a better way for evaluation of high-risk network operations.

Using NDT, operations causing potential network failures can be identified . NDT can also be utilized for optimization and verification of network policies and solutions for the further risk avoidance. After simulating/emulating a scenario, NDT can report the results back to the consumer. Based on results, The consumer can evaluate and verify possible network policies and solutions to minimize the impact of high-risk operations.

As an example, SLA degradation and failure of single node in mobile network can also be predicted using the NDT. When it is predicted that the network resources in the network domain are not enough to maintain the SLA or hardware resources failure in the future, the NDT results helps the consumer to act appropriately to avoid  network failure.

5.1.4	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- NFARP-01: | The NDT should have the capability allowing the consumer to request simulation/emulation of operations that could cause network failure. | NFARP |
| REQ- NFARP-02: | The NDT should have the capability to provide the simulation/emulation results including impact of operations. | NFARP |
| REQ- NFARP-03: | NDT should have a capability enabling the MnS consumer to configure the network scenario to be modelled for the consumer to evaluate a Network failure. | NFARP |

End of Second Changes