3GPP TSG-SA5 Meeting #160	S5-251471

Goteborg, SWEDEN 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0440 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.260 Charging principle for IMS DC application download charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2-CH |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The event-based charging for the IMS DC application download is a new charging scenario to support the monetization of IMS DC service on per application download event basis. The solution based on DCSF is recommended into normative work, per TR 28.851 solution #2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the charging principles and scenarios for IMS DC application download charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to charge the IMS DC service based on the application download event. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

Annex X (normative): Support event-based charging for IMS DC application download

X.1	General

The annex specifies the event-based charging for IMS DC application download, with the DCSF based converged charging architecture.

The duration-based charging for IMS DC application download refers to clause 5.1.15.

X.2	Architecture considerations

The high level IMS DC architecture is specified in clause 4.1.

The converged charging architecture reuse the IMS node converged charging architecture in clause 4.4, where the IMS Node refers to the DCSF in case of event based charging for IMS DC application download.

X.3	Charging principles and scenarios

X.3.1	General

The converged charging to support event-based charging for IMS DC application download may be performed by DCSF. The DCSF shall be able to perform converged charging by interacting with CHF, for charging data related to IMS DC application download event, based on PEC scenario specified in TS 32.290 [4].

X.3.2	Requirement

The following are high-level charging requirements for DCSF.

-	The DCSF shall support converged charging.

-	The DCSF shall collect and report charging information for IMS DC application download event.

X.3.3	Applicable triggers

Table X.3.3-1 summarizes the trigger conditions and their category which shall be supported by the DCSF when charging is active for the corresponding IMS DC application download functionality.

Table X.3.3-1: Default Trigger conditions in DCSF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Sent HTTP Response | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

X.3.3	Message flows

Figure X.3.3-1 depicts the charging procedure for IMS DC application download based on DCSF.

Figure X.3.3-1: PEC Charging Procedure for IMS DC application download

1. Originating UE initiate the bootstrap DC set up procedure, as specified in step 1-20 of Figure AC.7.1-1 TS 23.228 [201]. Boostrap DC has been established between originating/terminating MF and originating/terminating UE, as specified in step 21-24 of Figure AC.7.1-1 TS 23.228 [201].

2. In an established bootstrap DC, a UE may send application request message to MF using HTTP Get Request to download dedicated DC application(s).

The MF, acting as HTTP proxy, prepare the subscriber specific HTTP URL corresponding to the stream ID of the bootstrap DC and forward the request to the DCSF.

The DCSF, acting as HTTP web server, provide the subscriber specific HTTP URL with the requested DC application(s) in HTTP response to the UE through MF.

NOTE 1:	The detailed procedures to download DC application to the UE are out of scope as specified in TS 23.228 [201].

2ch-a. After DCSF sent the HTTP response to the UE, the DCSF sends Charging Data Request [Event] to CHF for this DC application download event.

2ch-b. The CHF creates the CDR for this DC application download event.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the DCSF.

X.4	Charging information

The Charging Data Request/Response and CDR definition reuse the data description for IMS converged charging specified in clause 6.4.

The IMS DC application download specific information is within the IMS charging information specified in clause 6.4.2.2.

End of Change