3GPP TSG-SA5 Meeting #160	S5-251511

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Ericsson

# Title:	Charging architecture considerations for CAPIF

# Document for:	Discussion

Agenda Item:	7.4.7

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs"

[2]	3GPP TR 28.849: "Study on charging aspects of Common API Framework for Northbound APIs (CAPIF) phase 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs	"

[4]	3GPP TS 32.240: "Charging Architecture and Principles"

[5]	3GPP TS 32.255: "5G data connectivity domain charging; Stage 2"

[6]	3GPP TS 32.254: "Exposure function Northbound Application Program Interfaces (APIs) charging"

[7]	3GPP TS 32.290: " 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[8]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

# 3	Rationale

## 3.1	Background

The current study TR 32.849 [2] has concluded that the CAPIF Core Function (CCF) would be the acting as CTF, however there are several reference points which all have different implications on what can be charged for and how the interaction between the CAPIF core function and the CHF may look.

Below is focused on the trusted domain but the same is applicable for the non-trusted i.e., the e-variant of the reference points, where applicable.

## 3.2	CAPIF-1

### 3.2.1	General

The CAPIF-1 reference point (TS 23.222 [1] clauses 6.4.2 and 6.4.3), which exists between the API invoker and the CAPIF core function, is used for the API invoker within the PLMN trust domain to discover service APIs, to authenticate and to get authorization.

The CAPIF-1 reference point supports:

-	Onboarding the new API invokers and offboarding API invokers.

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Mutual authentication between the API invoker and the CAPIF core function.

-	Providing authorization for the API invoker prior to accessing the service AP.

-	Providing authorization for the API invoker based on RNAA.

-	Discovering the service APIs information.

Where the main chargeable events are:

-	Onboarding the new API invokers and offboarding API invokers.

-	Discovering the service APIs information

In this case there are only two entities involved and that is the API Invoker and the CCF, and since the API Invoker may be outside the operator’s trusted domain the only place to trigger charging is the CCF.

### 3.2.2	CAPIF core function acting as CTF

In this case the CTF would be in the CAPIF core function and trigger based on the CAPIF-1 invocations. The below only shows onboarding with Post Event Charging (PEC), other services on CAPIF-1, e.g. discovery, would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure 3.2.2-1: API invoker onboarding PEC with CCF acting as CTF

Based on the flow in TS 23.222 [1] clause 8.1.

1.	The API Invoker triggers onboard API invoker request towards the CAPIF Core Function, for more details see TS 29.222 [3].

2.	After the CAPIF Core Function have performed the onboarding process it responds with an onboard API Invoker response, for more details see TS 29.222 [3].

3.	The CAPIF core function sends Charging Data Request [Event] to the CHF for reporting the charging information API invoker id and outcome of the onboarding, for more details see TS 32.290 [7].

4.	The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [7].

## 3.3	CAPIF-2

### 3.3.1	General

The CAPIF-2 reference point (TS 23.222 [1] clauses 6.4.4 and 6.4.5), which exists between the API invoker and the API exposing function belonging to the same trust domain, is used for the API invoker to communicate with the service APIs.

The CAPIF-2 reference point supports:

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Authorization verification for the API invoker upon accessing the service API.

-	Invocation of service APIs.

Where the main chargeable events are:

-	Invocation of service APIs.

### 3.3.2	CAPIF core function acting as CTF (ADF)

In this case the API Exposing Function in the API provider domain would act as a Charging Trigger Function (Accounting Metrics Collection) (CTF (AMC)) and the CAPIF core function would act as a Charging Trigger Function (Accounting Data Forwarding) (CTF ADF) and trigger based on the CAPIF-2 invocations. For AMC and ADF see TS 32.240 [4] annex D. The below only shows onboarding with Post Event Charging (PEC), other services on CAPIF-2 would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure 3.3.2-1: Invocation of service APIs PEC with CCF acting as CTF (ADF)

Based on the flow in TS 23.222 [1] clause 8.20.

1.	The API Invoker invokes an API towards the API Exposing Function in the API provider domain, for more details see TS 29.222 [3].

2.	After the execution of the service provided by the API invoked the API Exposing Function in the API provider domain responds with the result, for more details see TS 29.222 [3].

3.	Upon invocation of service API(s) from one more API invokers, the API Provider triggers an API invocation charging request to the CAPIF Core Function, for more details see TS 29.222 [3].

4.	The API Provider receives the API invocation charging response from the CAPIF Core Function, for more details see TS 29.222 [3].

5.	The CAPIF core function sends Charging Data Request [Event] to the CHF for reporting the charging information including API invoker id and result (e.g. failure or success) of the API invocation, for more details see TS 32.290 [7].

6.	The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [7].

### 3.3.3	API Exposing Function in the API provider domain acting as CTF

In this case the API Exposing Function in the API provider domain would act as a Charging Trigger Function (CTF) and trigger based on the CAPIF-2 invocations for API provider. The below only shows onboarding with Post Event Charging (PEC), other services on CAPIF-2 would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure 3.3.3-1: Invocation of service APIs PEC with CCF acting as CTF

Based on the flow in TS 23.222 [1] clause 8.20.

1.	The API Invoker invokes an API towards the API Exposing Function in the API provider domain, for more details see TS 29.222 [3].

2.	After the execution of the service provided by the API invoked the API Exposing Function in the API provider domain responds with the result, for more details see TS 29.222 [3].

3.	Optionally upon invocation of service API(s) from one more API invokers, the API Exposing Function in the API provider domain triggers an API invocation charging request, for more details see TS 29.222 [3].

4.	The API Exposing Function in the API provider domain receives the API invocation charging response from the CAPIF Core Function for more details see TS 29.222 [3].

5.	The API Exposing Function in the API provider domain sends Charging Data Request [Event] to the CHF for reporting the charging information including API invoker id and result (e.g. failure or success) of the API invocation, for more details see TS 32.290 [7].

6.	The CHF acknowledges by sending Charging Data Response [Event] to the API Exposing Function in the API provider domain, for more details see TS 32.290 [7].

### 3.3.4	CAPIF core function acting as CHF

In this case the API Exposing Function in the API provider domain act as a Charging Trigger Function (CTF) and CAPIF core function would act as an original CHF in an inter-CHF scenario, see TS 32.255 [5], the trigger based on the CAPIF-2 invocations for API provider and CAPIF-3 invocations for CAPIF core function. The below only shows onboarding with Post Event Charging (PEC), other services on CAPIF-2 would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure 3.3.4-1: Invocation of service APIs PEC with CCF acting as CHF

Based on the flow in TS 23.222 [1] clause 8.20.

1.	The API Invoker invokes an API towards the API Exposing Function in the API provider domain, for more details see TS 29.222 [3].

2.	After the execution of the service provided by the API invoked the API Exposing Function in the API provider domain responds with the result, for more details see TS 29.222 [3].

3.	The API Exposing Function in the API provider domain sends Charging Data Request [Event] to the CHF for reporting the charging information including API invoker id and result (e.g. failure or success) of the API invocation, for more details see TS 29.222 [3].

4.	The CAPIF core function acting as a CHF sends Charging Data Request [Event] using inter-CHF information to the CHF for reporting the charging information, for more details see TS 32.290 [7].

5.	The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [7].

6.	The CAPIF Core Function acting CHF acknowledges by sending Charging Data Response [Event] to the API Exposing Function in the API provider domain, for more details see TS 32.290 [7].

## 3.4	CAPIF-3

### 3.4.1	General

The CAPIF-3 reference point (TS 23.222 [1] clauses 6.4.6 and 6.4.9), which exists between the API exposing function and the CAPIF core function, is used for exercising access and policy related control for service API communications initiated by the API invoker.

The CAPIF-3 reference point supports:

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Providing authorization for the API invoker prior to accessing the service API.

-	Authorization verification for the API invoker upon accessing the service API.

-	Authorization verification for the API invoker based on RNAA.

-	Controlling the service API access based on PLMN operator configured policies.

-	Logging the service API invocations.

-	Charging the service API invocations.

There are no real chargeable events on this reference point, for the charging of API invocations see clause 3.3.

## 3.5	CAPIF-4

### 3.5.1	General

The CAPIF-4 reference point (TS 23.222 [1] clauses 6.4.7 and 6.4.10), which exists between the API publishing function and the CAPIF core function, is used for publishing the service API information.

The CAPIF-4 reference point supports:

-	Publishing the service APIs information by the API publishing function.

Where the main chargeable event is:

-	Publishing the service APIs information.

### 3.5.2	CAPIF core function acting as CTF

In this case the CTF would be in the CAPIF core function and trigger based on the CAPIF-1 invocations. The below only shows onboarding with Post Event Charging (PEC), other services on CAPIF-4 would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure 3.5.2-1: Publishing the service API PEC with CCF acting as CTF

Based on the flow in TS 23.222 [1] clause 8.3.

1.	The API Publishing Function in the API provider domain sends a service API publish request to the CAPIF Core Function, for more details see TS 29.222 [3].

2.	After the CAPIF core function have performed the publishing process the CAPIF Core Function provides a service API publish response to the API Publishing Function in the API provider domain, for more details see TS 29.222 [3].

3.	The CAPIF Core Function sends Charging Data Request [Event] to the CHF for reporting the charging information including API provider id and outcome of the publishing, for more details see TS 32.290 [7].

4.	The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [7].

## 3.6	CAPIF-5

### 3.6.1	General

The CAPIF-5 reference point (TS 23.222 [1] clauses 6.4.8 and 6.4.11), which exists between the API management function and the CAPIF core function, is used for management of service API, API invoker and API provider domain function information.

The CAPIF-5 reference point supports:

-	Accessing the service API invocation logs by the API management function.

-	Enabling the API management function to monitor the events reported due to the service APIs invocations.

-	Onboarding new API invokers by provisioning the API invoker information at the CAPIF core function, requesting explicit grant of new API invokers onboarding and confirming onboarding success.

-	Offboarding API invokers.

-	Enabling the API management function to configure policies at the CAPIF core function e.g. service API invocation throttling, blocking API invocation for certain duration.

-	Enabling the API provider domain to monitor the status of service APIs (e.g. pilot or live status, start or stop status of service API).

-	Registering API provider domain functions on the CAPIF core function.

-	Update of the registration information of API provider domain functions on the CAPIF core function.

NOTE 1:	The API invoker onboarding/offboarding over CAPIF-5 is out of the scope of the current release.

Where the main chargeable events are:

-	Accessing the service API invocation logs.

-	Onboarding new API invokers by provisioning the API invoker information at the CAPIF core function, requesting explicit grant of new API invokers onboarding and confirming onboarding success.

-	Offboarding API invokers.

-	Monitor the events reported due to the service APIs invocations.

-	Configure policies at the CAPIF core function e.g. service API invocation throttling, blocking API invocation for certain duration.

-	Monitor the status of service APIs (e.g. pilot or live status, start or stop status of service API).

-	Registering API provider domain functions.

-	Update of the registration information of API provider domain functions on the CAPIF core function.

The charging of these would be like the CAPIF-4 since these are triggered by the API Management Function in the API provider domain, like the publish request. See clause 3.5.2 the steps 1 and 2 would have other CAPIF core services.

## 3.7	CAPIF-6

### 3.7.1	General

The CAPIF-6 reference point (TS 23.222 [1] clauses 6.4.14 and 6.4.15), which exists between the CAPIF core functions instances within the same trust domain.

The CAPIF-6 reference point supports:

-	Publishing the service APIs information.

-	Discovering the service APIs information.

Where the main chargeable events are:

-	Publishing the service APIs information.

-	Discovering the service APIs information.

### 3.7.2	CAPIF core function acting as CTF

In this case the CTF would be in the CAPIF core function and trigger based on the CAPIF-6 invocations. The below only shows onboarding with Post Event Charging (PEC), other services on CAPIF-4 would be handled similarly, also Immediate Event Charging (IEC) and Event Charging with Unit Reservation. (ECUR) could be supported.

Figure 3.5.2-1: Publishing the service API PEC with CCF acting as CTF

Based on the flow in TS 23.222 [1] clause 8.25.

1.	Based on the shareable information for the service API or the service API category information, the CAPIF Core Function A (CCF-A) determines to publish the service API or the service API category information to the CAPIF Core Function B (CCF-B), for more details see TS 29.222 [3].

2.	After the CCF-B have performed the publishing process the CCF-B provides an interconnection API publish response to the CCF-A, for more details see TS 29.222 [3].

3.	The CCF-B sends Charging Data Request [Event] to the CHF for reporting the charging information including API provider id and outcome of the publishing, for more details see TS 32.290 [7].

4.	The CHF acknowledges by sending Charging Data Response [Event] to the CCF-B, for more details see TS 32.290 [7].

## 3.8	CAPIF-7

### 3.8.1	General

The CAPIF-7 reference point (TS 23.222 [1] clauses 6.4.12 and 6.4.13), which exists between the API exposing functions belonging to the same trust domain, is used for the forwarding or routing of the API invoker's service API invocation from one API exposing function to the other API exposing function deployed in the PLMN trust domain.

The CAPIF-7 reference point supports all the functions of CAPIF-2.

The CAPIF-7 reference point supports invocation of service APIs originated by the API invoker using CAPIF-2.

NOTE:	The aspects related to the specific service API invocation in reference point CAPIF-7 are out of scope of the present document.

For chargeable events see CAPIF-2.

## 3.9	CAPIF-8

### 3.9.1	General

The CAPIF-8 reference point (TS 23.222 [1] clauses 6.4.16), which exists between the CAPIF core function and the resource owner function.

The CAPIF-8 reference point supports:

-	Providing authorization for resource access.

-	Managing and revoking the provided authorization.

NOTE:	The functionalities over CAPIF-8 is FFS and out of scope of the current release of the specification.

## 3.10	Evaluation

For CAPIF-1, CAPIF-4, CAPIF-5, and CAPIF-6 the solution and information needed for charging is straight forward i.e., the CAPIF Core Function will act as an CTF and trigger charging for the services it provides, and that can be to API invokers (CAPIF-1), API Publishing Function in the API provider domain (CAPIF-4), API Management Function in the API provider domain (CAPIF-5) and to peer CAPIF Core Function instances (CAPIF-6).

For CAPIF-3 there is no real need to have any charging, but if it would be required it could reuse the same solution as for CAPIF-4.

For CAPIF-2 there are multiple solutions, and could be used for in different contexts:

-	CAPIF core function acting as CTF (ADF) is the most straight forward solution and would allow CAPIF to specify the CAPIF-3 with limited impact on the Nchf, compare the solution for ProSe and the handling of PC5 TS 23.304 [8].

-	CAPIF core function acting as CTF is like CAPIF core function acting as CTF (ADF), it however doesn’t exclude allowing the API Provider acting as a CTF. Having two entities acting a CTF for the same chargeable event may cause issues.

-	CAPIF core function acting as CHF is based in the inter-CHF feature defined in TS 32.255 [4] where the CAPIF core function would forward Charging Data Requests as specified in TS 32.254 [5] from the API Exposing Function in the API provider domain and include inter-CHF information.

For CAPIF-7 and CAPIF-8 there is currently little need for charging since these are outside the scope of the current CAPIF specification.

In the below table a summary of the alternatives are provided where the main alternatives for acting like a CTF are CAPIF Core Function (CCF) and the API Exposing Function (AEF) in the API provider domain.

Table 3.10.1:

| Reference point | Charging trigger function | Comment |
| --- | --- | --- |
| CAPIF-1 | CCF |  |
| CAPIF-2 | - CCF - AEF - CCF (ADF), AEF (ACM) | Three alternatives. |
| CAPIF-3 | None | If required CCF could be used |
| CAPIF-4 | CCF |  |
| CAPIF-5 | CCF |  |
| CAPIF-6 | CCF |  |
| CAPIF-7 | None | If required CAPIF-2 solution could be reused |
| CAPIF-8 | None | If required CCF could be used |

## 3.11	Observations

Observation 1: Having the charging trigger in the CAPIF Core Function for services provided by the CAPIF core function is the only way forward.

Observation 2: For charging of API invocation having the CAPIF core function acting as CTF (ADF) would reuse CAPIF-3 services, it is however hard to exclude the solution where an API Exposing Function in the API provider domain act as a CTF and communicates directly with the CHF i.e., the NEF scenario.

Observation 3: CAPIF-3, CAPIF-7, and CAPIF-8 doesn’t require any charging specification at this moment.

# 4	Detailed proposal

Proposal 1: Specify solutions where CAPIF Core Function may act as either a full CTF or an CTF (ADF)

Proposal 2: Add clauses 3.1 to 3.10 to the current study.

Annex A (informative):
PlantUML

# A.1	Flows

## A.1.1	Figure 3.2.2-1

@startuml

hide footbox

participant APP as "API\nInvoker"

participant CCF as "CAPIF Core\nFuntion\n(CTF)"

participant CHF as "CHF"

autonumber

APP -> CCF : CAPIF-1 [Onboarding of API Invoker]

APP <- CCF : CAPIF-1 [Onboarding response]

CCF -> CHF : Charging Data request [Event]

CCF <- CHF : Charging Data response [Event]

@enduml

## A.1.2	Figure 3.3.2-1

@startuml

hide footbox

participant APP as "API\nInvoker"

participant NEF as "API\nProvider\nCTF (AMC)"

participant CCF as "CAPIF Core\nFuntion\nCTF (ADF)"

participant CHF as "CHF"

autonumber

APP -> NEF : CAPIF-2 [Invocation of service API]

APP <- NEF : CAPIF-2 [Invocation response]

NEF -> CCF : CAPIF-3 [API invocation charging request]

NEF <- CCF : CAPIF-3 [API invocation charging response]

CCF -> CHF : Charging Data request [Event]

CCF <- CHF : Charging Data response [Event]

@enduml

## A.1.3	Figure 3.3.3-1

@startuml

hide footbox

participant APP as "API\nInvoker"

participant NEF as "API\nProvider\n(CTF)"

participant CCF as "CAPIF Core\nFuntion"

participant CHF as "CHF"

autonumber

APP -> NEF : CAPIF-2 [Invocation of service API]

APP <- NEF : CAPIF-2 [Invocation response]

NEF --> CCF : CAPIF-3 Charging information request

NEF <-- CCF : CAPIF-3 Charging information respons

NEF -> CHF : Charging Data request [Event]

NEF <- CHF : Charging Data response [Event]

@enduml

## A.1.4	Figure 3.3.4-1

@startuml

hide footbox

participant APP as "API\nInvoker"

participant NEF as "API\nProvider\n(CTF)"

participant CCF as "CAPIF Core\nFuntion\n(CHF)"

participant CHF as "CHF"

autonumber

APP -> NEF : CAPIF-2 [Invocation of service API]

APP <- NEF : CAPIF-2 [Invocation response]

NEF -> CCF : Charging Data request [Event]

CCF -> CHF : Charging Data request [Event]

CCF <- CHF : Charging Data response [Event]

NEF <- CCF : Charging Data response [Event]

@enduml

## A.1.6	Figure 3.5.2-1

@startuml

hide footbox

participant NEF as "API\nProvider"

participant CCF as "CAPIF Core\nFuntion\n(CTF)"

participant CHF as "CHF"

autonumber

NEF -> CCF : CAPIF-4 [Service API publish request]

NEF <- CCF : CAPIF-4 [Service API publish response]

CCF -> CHF : Charging Data request [Event]

CCF <- CHF : Charging Data response [Event]

@enduml

## A.1.7	Figure 3.7.2-1

@startuml

hide footbox

participant CCF_A as "CAPIF Core\nFunction A"

participant CCF_B as "CAPIF Core\nFuntion B\n(CTF)"

participant CHF as "CHF"

autonumber

CCF_A -> CCF_B : CAPIF-6 [Interconnection API publish request]

CCF_A <- CCF_B : CAPIF-6 [Interconnection API publish response]

CCF_B -> CHF : Charging Data request [Event]

CCF_B <- CHF : Charging Data response [Event]

@enduml