3GPP TSG-SA5 Meeting #160	S5-251648

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei

Title:	Pseudo-CR on TS 28.561 Update use case for NDT support analysis of failure events including signaling storm

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.1.0

Work Item:	NDT

Comments

This contribution proposes to update the use case in clause 5.2.2.3 and add the requirement.

Proposed Changes

* * * First Change * * * *

5.2.2.3	Support analysis of failure events including Signaling storm

A failure event refers to the situation where users in the network are completely unable to get service form the network. An example failure events is a signaling storm, which is the situation where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm.

When a signaling storm occurs on the network, Network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points during a signaling storm. The NDT can be used to validate whether the proposed solution can resolve the signaling storm issue.

The NDT can also model network behaviors and provide information on potential impact of network failures including on surges in signaling requests on the network. For a signaling storm, for example, the information can enable the MnS consumer to evaluate the network’s ability to defend against a possible signaling storm and identify the optimal flow control parameters for each signaling impact point.

* * * Next Change * * * *

5.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTAUTO-AUTOSS-01: | The 3GPP management system should support a capability to model (either using emulation method or simulation method) the behaviour of the network facing the signaling storm. | Support analysis of failure events including Signaling storm(clause 5.2.2.3) |
| REQ-NDTAUTO-AUTOSS-02: | The 3GPP management system should support a capability for an authorized NDT MnS consumer to define a network configuration to be modelled for checking the network response to a signaling storm from NDT. | Support analysis of failure events including Signaling storm(clause 5.2.2.3) |
| REQ-NDTAUTO-AUTOSS-03: | The 3GPP management system should support a capability to report potential behaviour of a network facing a failure event including a signaling storm. | Support analysis of failure events including Signaling storm(clause 5.2.2.3) |

* * * End of Changes * * * *